# Logical Netlist

## kh

## Block Diagram

## Component Instances

| Ref | Part Number | Component |
| --- | --- | --- |
| U1 | HMC1119LP4DE | Wideband_LNA |
| U2 | HMC698LP4 | Digital_VGA |
| U3 | HMC1051LP4BE | RF_Mixer |
| U4 | ADL5541 | IF_Amplifier |
| U5 | ADC12DJ5200RF | High_Speed_ADC |
| U6 | LMK04828BKNQ | Clock_Generator |
| U7 | LTC2975 | Power_Controller |
| U8 | LTM4644IY | 5V_DCDC_Buck |
| U9 | LTM4644IY | 3V3_DCDC_Buck |
| U10 | LTM4625 | 1V8_DCDC_Buck |
| U11 | LTM4625 | 1V2_DCDC_Buck |
| J1 | SMA-J-P-H-ST-EM1 | RF_Input_Connector |
| J2 | SMA-J-P-H-ST-EM1 | LO_Input_Connector |
| J3 | SAMTEC_ERF8-140-01 | Digital_Output_Connector |
| J4 | Molex_5034801200 | Power_Input_Connector |
| T1 | EQL-U1-300+ | RF_Balun_10to15GHz |
| T2 | BALH-0006SMG | LO_Balun_6to26GHz |
| T3 | TCM1-63AX+ | IF_Balun_DCto6GHz |
| F1 | BP10S15G-01A | RF_Bandpass_Filter |
| F2 | LFCN-6000+ | IF_Antialias_Filter |
| L1 | BLM15HB121SN1D | RF_Chip_Bead |
| L2 | BLM15HB121SN1D | RF_Chip_Bead |
| L3 | BLM18PG471SN1D | Power_Chip_Bead |
| L4 | BLM18PG471SN1D | Power_Chip_Bead |
| L5 | BLM18PG471SN1D | Power_Chip_Bead |
| L6 | BLM18PG471SN1D | Power_Chip_Bead |
| L7 | BLM18PG471SN1D | Power_Chip_Bead |
| C1 | GQM2195C2E100JB12D | RF_Input_Coupling_Cap |
| C2 | GQM2195C2E100JB12D | RF_Coupling_Cap |
| C3 | GQM2195C2E100JB12D | RF_Coupling_Cap |
| C4 | GQM2195C2E100JB12D | RF_Coupling_Cap |
| C5 | GQM2195C2E270JB12D | IF_Coupling_Cap |
| C6 | GQM2195C2E270JB12D | IF_Coupling_Cap |
| C10 | GJM1555C1H101JB01D | Decoupling_Cap_100pF |
| C11 | GJM1555C1H101JB01D | Decoupling_Cap_100pF |
| C12 | GJM1555C1H101JB01D | Decoupling_Cap_100pF |
| C13 | GJM1555C1H101JB01D | Decoupling_Cap_100pF |
| C14 | GJM1555C1H101JB01D | Decoupling_Cap_100pF |
| C15 | GJM1555C1H101JB01D | Decoupling_Cap_100pF |
| C20 | GRM32ER71C106KE15L | Bulk_Cap_10uF |
| C21 | GRM32ER71C106KE15L | Bulk_Cap_10uF |
| C22 | GRM32ER71C106KE15L | Bulk_Cap_10uF |
| C23 | GRM32ER71C106KE15L | Bulk_Cap_10uF |
| C24 | GRM32ER71C106KE15L | Bulk_Cap_10uF |
| C25 | GRM32ER71C106KE15L | Bulk_Cap_10uF |
| C26 | GRM32ER71C106KE15L | Bulk_Cap_10uF |
| C30 | GRM155R71H103KA01D | Decoupling_Cap_0u01F |
| C31 | GRM155R71H103KA01D | Decoupling_Cap_0u01F |
| C32 | GRM155R71H103KA01D | Decoupling_Cap_0u01F |
| C33 | GRM155R71H103KA01D | Decoupling_Cap_0u01F |
| C34 | GRM155R71H103KA01D | Decoupling_Cap_0u01F |
| C35 | GRM155R71H103KA01D | Decoupling_Cap_0u01F |
| C36 | GRM155R71H103KA01D | Decoupling_Cap_0u01F |
| R1 | ERA-3AEB502V | RF_Input_Termination |
| R2 | ERA-3AEB301V | IF_Gain_Resistor |
| R3 | ERA-3AEB100V | ADC_Input_Termination |
| R4 | ERA-3AEB100V | ADC_Input_Termination |
| R10 | ERA-3AEB103V | SPI_Pullup |
| R11 | ERA-3AEB103V | SPI_Pullup |
| R12 | ERA-3AEB103V | SPI_Pullup |
| R13 | ERA-3AEB103V | Reset_Pullup |
| R14 | ERA-3AEB103V | I2C_Pullup |
| R15 | ERA-3AEB103V | I2C_Pullup |

## Pin-to-Pin Connections

| Net | From | Pin | To | Pin | Type |
| --- | --- | --- | --- | --- | --- |
| RF_IN_PATH | J1 | SIG | C1 | 1 | rf |
| RF_IN_PATH | C1 | 2 | F1 | IN | rf |
| RF_FILTERED | F1 | OUT | T1 | 1 | rf |
| RF_BALUN_P | T1 | 2 | U1 | RFIN_P | rf |
| RF_BALUN_N | T1 | 3 | U1 | RFIN_N | rf |
| LNA_OUT_P | U1 | RFOUT_P | C2 | 1 | rf |
| LNA_OUT_N | U1 | RFOUT_N | C3 | 1 | rf |
| VGA_IN_P | C2 | 2 | U2 | IN_P | rf |
| VGA_IN_N | C3 | 2 | U2 | IN_N | rf |
| VGA_OUT_P | U2 | OUT_P | C4 | 1 | rf |
| VGA_OUT_N | U2 | OUT_N | U3 | RF_P | rf |
| MIXER_IF_P | U3 | IF_P | T3 | 1 | if |
| MIXER_IF_N | U3 | IF_N | T3 | 3 | if |
| IF_SINGLE_ENDED | T3 | 2 | F2 | IN | if |
| IF_FILTERED | F2 | OUT | C5 | 1 | if |
| IF_AMP_IN | C5 | 2 | U4 | IN | if |
| IF_AMP_OUT | U4 | OUT | C6 | 1 | if |
| ADC_IN_P | C6 | 2 | U5 | AIN_P | analog |
| ADC_IN_N | R3 | 1 | U5 | AIN_N | analog |
| LO_IN_PATH | J2 | SIG | T2 | 1 | rf |
| LO_BALUN_P | T2 | 2 | U3 | LO_P | rf |
| LO_BALUN_N | T2 | 3 | U3 | LO_N | rf |
| CLK_ADC_P | U6 | CLKOUT0_P | U5 | CLKIN_P | clock |
| CLK_ADC_N | U6 | CLKOUT0_N | U5 | CLKIN_N | clock |
| SYSREF_P | U6 | SYNC_P | U5 | SYNC_P | clock |
| SYSREF_N | U6 | SYNC_N | U5 | SYNC_N | clock |
| JESD204B_CKP | U5 | CXP | J3 | JESD_CKP | digital |
| JESD204B_CKN | U5 | CXN | J3 | JESD_CKN | digital |
| JESD204B_D0_P | U5 | D0P | J3 | JESD_D0P | digital |
| JESD204B_D0_N | U5 | D0N | J3 | JESD_D0N | digital |
| JESD204B_D1_P | U5 | D1P | J3 | JESD_D1P | digital |
| JESD204B_D1_N | U5 | D1N | J3 | JESD_D1N | digital |
| JESD204B_D2_P | U5 | D2P | J3 | JESD_D2P | digital |
| JESD204B_D2_N | U5 | D2N | J3 | JESD_D2N | digital |
| JESD204B_D3_P | U5 | D3P | J3 | JESD_D3P | digital |
| JESD204B_D3_N | U5 | D3N | J3 | JESD_D3N | digital |
| SPI_SCLK | U7 | GPIO1 | U2 | SCLK | digital |
| SPI_SCLK | U7 | GPIO1 | U5 | SCLK | digital |
| SPI_SDIO | U7 | GPIO2 | U2 | SDIO | digital |
| SPI_SDIO | U7 | GPIO2 | U5 | SDIO | digital |
| SPI_CSN_VGA | U7 | GPIO3 | U2 | CSN | digital |
| SPI_CSN_ADC | U7 | GPIO4 | U5 | CSN | digital |
| I2C_SCL | U7 | GPIO5 | U6 | SCL | digital |
| I2C_SDA | U7 | GPIO6 | U6 | SDA | digital |
| 5V_REG_OUT | U8 | VOUT1 | L3 | 1 | power |
| 5V_LNA | L3 | 2 | L1 | 1 | power |
| 5V_LNA_FILTERED | L1 | 2 | U1 | VCC | power |
| 5V_VGA | L3 | 2 | L2 | 1 | power |
| 5V_VGA_FILTERED | L2 | 2 | U2 | VCC | power |
| 5V_MIXER | L3 | 2 | U3 | VCC | power |
| 5V_IF_AMP | L3 | 2 | U4 | VCC | power |
| 3V3_REG_OUT | U9 | VOUT2 | L4 | 1 | power |
| 3V3_CLK | L4 | 2 | U6 | VCC_IO | power |
| 3V3_PWR_CTRL | L4 | 2 | U7 | VCC | power |
| 1V8_REG_OUT | U10 | VOUT | L5 | 1 | power |
| 1V8_ADC_CORE | L5 | 2 | U5 | VCC | power |
| 1V2_REG_OUT | U11 | VOUT | L6 | 1 | power |
| 1V2_ADC_IO | L6 | 2 | U5 | VCC_IO | power |
| VIN_12V | J4 | 1 | L7 | 1 | power |
| VIN_FILTERED | L7 | 2 | U8 | VIN | power |
| VIN_FILTERED | L7 | 2 | U9 | VIN | power |
| GND | U1 | GND | U2 | GND | ground |
| GND | U2 | GND | U3 | GND | ground |
| GND | U3 | GND | U4 | GND | ground |
| GND | U4 | GND | U5 | GND | ground |
| GND | U5 | GND | J4 | 2 | ground |
| 5V_DECOUPLE | L3 | 2 | C20 | 1 | power |
| 5V_DECOUPLE | C20 | 1 | C10 | 1 | power |
| 3V3_DECOUPLE | L4 | 2 | C21 | 1 | power |
| 3V3_DECOUPLE | C21 | 1 | C11 | 1 | power |
| 1V8_DECOUPLE | L5 | 2 | C22 | 1 | power |
| 1V8_DECOUPLE | C22 | 1 | C12 | 1 | power |
| 1V2_DECOUPLE | L6 | 2 | C23 | 1 | power |
| 1V2_DECOUPLE | C23 | 1 | C13 | 1 | power |

## Net Connection List

| Net Name | Reference Designator - Pin No. |
| --- | --- |
| 1V2_ADC_IO | L6 - 2, U5 - VCC_IO |
| 1V2_DECOUPLE | L6 - 2, C23 - 1, C13 - 1 |
| 1V2_REG_OUT | U11 - VOUT, L6 - 1 |
| 1V8_ADC_CORE | L5 - 2, U5 - VCC |
| 1V8_DECOUPLE | L5 - 2, C22 - 1, C12 - 1 |
| 1V8_REG_OUT | U10 - VOUT, L5 - 1 |
| 3V3_CLK | L4 - 2, U6 - VCC_IO |
| 3V3_DECOUPLE | L4 - 2, C21 - 1, C11 - 1 |
| 3V3_PWR_CTRL | L4 - 2, U7 - VCC |
| 3V3_REG_OUT | U9 - VOUT2, L4 - 1 |
| 5V_DECOUPLE | L3 - 2, C20 - 1, C10 - 1 |
| 5V_IF_AMP | L3 - 2, U4 - VCC |
| 5V_LNA | L3 - 2, L1 - 1 |
| 5V_LNA_FILTERED | L1 - 2, U1 - VCC |
| 5V_MIXER | L3 - 2, U3 - VCC |
| 5V_REG_OUT | U8 - VOUT1, L3 - 1 |
| 5V_VGA | L3 - 2, L2 - 1 |
| 5V_VGA_FILTERED | L2 - 2, U2 - VCC |
| ADC_IN_N | R3 - 1, U5 - AIN_N |
| ADC_IN_P | C6 - 2, U5 - AIN_P |
| CLK_ADC_N | U6 - CLKOUT0_N, U5 - CLKIN_N |
| CLK_ADC_P | U6 - CLKOUT0_P, U5 - CLKIN_P |
| GND | U1 - GND, U2 - GND, U3 - GND, U4 - GND, U5 - GND, J4 - 2 |
| I2C_SCL | U7 - GPIO5, U6 - SCL |
| I2C_SDA | U7 - GPIO6, U6 - SDA |
| IF_AMP_IN | C5 - 2, U4 - IN |
| IF_AMP_OUT | U4 - OUT, C6 - 1 |
| IF_FILTERED | F2 - OUT, C5 - 1 |
| IF_SINGLE_ENDED | T3 - 2, F2 - IN |
| JESD204B_CKN | U5 - CXN, J3 - JESD_CKN |
| JESD204B_CKP | U5 - CXP, J3 - JESD_CKP |
| JESD204B_D0_N | U5 - D0N, J3 - JESD_D0N |
| JESD204B_D0_P | U5 - D0P, J3 - JESD_D0P |
| JESD204B_D1_N | U5 - D1N, J3 - JESD_D1N |
| JESD204B_D1_P | U5 - D1P, J3 - JESD_D1P |
| JESD204B_D2_N | U5 - D2N, J3 - JESD_D2N |
| JESD204B_D2_P | U5 - D2P, J3 - JESD_D2P |
| JESD204B_D3_N | U5 - D3N, J3 - JESD_D3N |
| JESD204B_D3_P | U5 - D3P, J3 - JESD_D3P |
| LNA_OUT_N | U1 - RFOUT_N, C3 - 1 |
| LNA_OUT_P | U1 - RFOUT_P, C2 - 1 |
| LO_BALUN_N | T2 - 3, U3 - LO_N |
| LO_BALUN_P | T2 - 2, U3 - LO_P |
| LO_IN_PATH | J2 - SIG, T2 - 1 |
| MIXER_IF_N | U3 - IF_N, T3 - 3 |
| MIXER_IF_P | U3 - IF_P, T3 - 1 |
| RF_BALUN_N | T1 - 3, U1 - RFIN_N |
| RF_BALUN_P | T1 - 2, U1 - RFIN_P |
| RF_FILTERED | F1 - OUT, T1 - 1 |
| RF_IN_PATH | J1 - SIG, C1 - 1, C1 - 2, F1 - IN |
| SPI_CSN_ADC | U7 - GPIO4, U5 - CSN |
| SPI_CSN_VGA | U7 - GPIO3, U2 - CSN |
| SPI_SCLK | U7 - GPIO1, U2 - SCLK, U5 - SCLK |
| SPI_SDIO | U7 - GPIO2, U2 - SDIO, U5 - SDIO |
| SYSREF_N | U6 - SYNC_N, U5 - SYNC_N |
| SYSREF_P | U6 - SYNC_P, U5 - SYNC_P |
| VGA_IN_N | C3 - 2, U2 - IN_N |
| VGA_IN_P | C2 - 2, U2 - IN_P |
| VGA_OUT_N | U2 - OUT_N, U3 - RF_P |
| VGA_OUT_P | U2 - OUT_P, C4 - 1 |
| VIN_12V | J4 - 1, L7 - 1 |
| VIN_FILTERED | L7 - 2, U8 - VIN, U9 - VIN |

## Validation Notes

RF Layout Critical: 10-15 GHz signals require controlled impedance (50 ohm) transmission lines on Rogers 4350B or similar high-frequency laminate with dielectric constant ~3.66. Keep RF traces short, avoid vias in RF path.

LNA and VGA are 5V devices - ensure proper decoupling with C10/C11 (100pF) and C20/C21 (10uF) as close as possible to supply pins. Ferrite beads L1/L2 help isolate RF noise.

ADC requires ultra-clean 1.8V core supply and 1.2V IO supply - use separate regulator channels U10/U11 with dedicated filtering L5/L6 and C22/C23 decoupling network.

JESD204B interface at up to 12 Gbps requires precise impedance matching (100 ohm differential) and length matching between P/N pairs to within 5 mils.

LO drive requirement: Mixer requires +10 dBm LO drive - ensure LO source can provide this or add LO amplifier stage if needed.

Thermal considerations: ADC dissipates 2.5W, LNA 0.5W, VGA 0.6W, Mixer 0.4W, IF Amp 0.3W - total ~4.3W in signal chain. Use thermal vias under power pads and consider copper pour heatsinking.

Power sequencing: ADC requires 1.2V IO before 1.8V core for proper operation. Implement sequencing in U7 power controller.

Ground strategy: Use unified ground plane but partition RF and digital sections. Place ground stitching vias along partition boundaries.

VGA gain control: 6-bit parallel interface requires pullup resistors R10-R12 to prevent floating inputs during power-up.

IF filter F2 provides anti-aliasing before ADC - ensure cutoff frequency is set appropriately for selected IF frequency (recommend 500 MHz or less).

Clock jitter: LMK04828 provides low-jitter clock - critical for ADC SNR performance. Use separate low-noise LDO for clock IC supply.

Input protection: Consider adding ESD protection diode at J1 input for industrial environment (-40 to +85C operation).

RF baluns T1/T2 and T3 are critical for single-ended to differential conversion - select appropriate frequency range rated components.

Bypass capacitors C30-C36 (0.01uF) placed very close to each IC supply pin for high-frequency decoupling.

Test points recommended at key nodes: RF input to mixer, IF output, ADC input, and JESD204B outputs for debugging.