# Register Description Table (RDT)

## kh

> **Total registers:** 54

**kh Wideband RF Receiver Module Register Map**

44 registers mapped across 11 functional groups (0x000-0x0A00)

UART command interface at 115200 bps with single/bulk read-write protocols

RF front-end control: HMC1119 LNA, HMC698LP4 VGA (6-bit gain), HMC1051 Mixer

JESD204B link control for ADC12DJ5200RF (5.2 GSPS)

Power monitoring via LTC2975 (5V, 3.3V, 1.8V, 1.2V rails)

Clock generation via LMK04828BKNQ with PLL lock monitoring

512 Mb Configuration Flash (S25FL512S) and EEPROM for calibration storage

20-step initialization sequence spanning power-on self-test through RF enable

## Register Address Decoding

The 16-bit UART register address is decoded as follows:

| Bit(s) | Field | Description |
| --- | --- | --- |
| [15] | R/W# | 1 = Read operation, 0 = Write operation |
| [14] | Reserved | Must be 0 |
| [13:12] | Reserved | Must be 0 |
| [11:8] | BASE_ADDR[3:0] | Functional group selector |
| [7:0] | OFFSET[7:0] | Register offset within group |

### Base Address Map

| BASE[3:0] | Address Range | Functional Group |
| --- | --- | --- |
| 0x0 | 0x0000–0x00FF | Board Information |
| 0x1 | 0x0100–0x01FF | Communication & Interface |
| 0x2 | 0x0200–0x02FF | ADC / Supply Monitoring |
| 0x3 | 0x0300–0x03FF | Temperature & Health |
| 0x4 | 0x0400–0x04FF | PLL / Clock Configuration |
| 0x5 | 0x0500–0x05FF | EEPROM / NV Storage |
| 0x6 | 0x0600–0x06FF | Configuration Flash |
| 0x7 | 0x0700–0x07FF | RF / Phase Control |
| 0x8 | 0x0800–0x08FF | GPIO / Control |
| 0x9 | 0x0900–0x09FF | DAC / Output |

## UART Frame Formats

### Single Register Write

### Single Register Read

### Bulk Write (N consecutive registers)

### Bulk Read (N consecutive registers)

## Register Definitions

| Address | Register Name | Access | Reset | Description |
| --- | --- | --- | --- | --- |
| `0x0000` | `BOARD_ID` | — | `0x4B48` | Board identification code - kh Wideband RF Receiver unique identifier |
| `0x0001` | `BOARD_VERSION` | — | `0x01` | Hardware version number |
| `0x0002` | `BOARD_TYPE_ID` | — | `0x5246` | Board type identifier |
| `0x0003` | `SCRATCHPAD` | — | `0x0000` | Scratchpad register for RAM/CPU connectivity test |
| `0x0010` | `MCS_VERSION_MAJOR` | — | `0x01` | FPGA firmware major version number |
| `0x0011` | `MCS_VERSION_MINOR` | — | `0x00` | FPGA firmware minor version number |
| `0x0012` | `BUILD_DATE` | — | `0x2415` | FPGA build date in packed BCD format (YYYYMMDD) |
| `0x0100` | `UART_BAUD_DIV` | — | `0x0036` | UART baud rate divisor (115200 bps default) |
| `0x0101` | `UART_CTRL` | — | `0x03` | UART control register |
| `0x0102` | `UART_STATUS` | — | `0x00` | UART status flags (read-clear) |
| `0x0103` | `UART_TX_COUNT` | — | `0x00` | TX FIFO byte count |
| `0x0104` | `UART_RX_COUNT` | — | `0x00` | RX FIFO byte count |
| `0x0200` | `ADC_CTRL` | — | `0x00` | ADC control for power monitoring |
| `0x0201` | `ADC_STATUS` | — | `0x00` | ADC status flags |
| `0x0210` | `VCC_5V_RAW` | — | `0x0000` | 5V rail ADC raw count |
| `0x0211` | `VCC_3V3_RAW` | — | `0x0000` | 3.3V rail ADC raw count |
| `0x0212` | `VCC_2V5_RAW` | — | `0x0000` | 2.5V rail ADC raw count |
| `0x0213` | `VCC_1V8_RAW` | — | `0x0000` | 1.8V rail ADC raw count |
| `0x0214` | `VCC_1V2_RAW` | — | `0x0000` | 1.2V rail ADC raw count (FPGA Core) |
| `0x0218` | `ICC_5V_RAW` | — | `0x0000` | 5V rail current sense ADC |
| `0x0219` | `ICC_3V3_RAW` | — | `0x0000` | 3.3V rail current sense ADC |
| `0x0300` | `TEMP_LOCAL` | — | `0x0000` | Local FPGA die temperature |
| `0x0301` | `TEMP_REMOTE1` | — | `0x0000` | Remote temperature sensor 1 (via LTC2975) |
| `0x0302` | `TEMP_REMOTE2` | — | `0x0000` | Remote temperature sensor 2 |
| `0x0308` | `TEMP_ALERT_HIGH` | — | `0x0190` | Over-temperature alert threshold |
| `0x0309` | `TEMP_ALERT_LOW` | — | `0x0288` | Under-temperature alert threshold |
| `0x030F` | `HEALTH_STATUS` | — | `0x00` | System health status flags |
| `0x0400` | `PLL_CTRL` | — | `0x00` | PLL/LMK04828 control register |
| `0x0401` | `PLL_STATUS` | — | `0x00` | PLL status flags |
| `0x0402` | `PLL_N_DIV` | — | `0x0064` | PLL N divider value |
| `0x0403` | `PLL_R_DIV` | — | `0x01` | PLL R divider value |
| `0x0410` | `CLK_ENABLE` | — | `0x00` | Clock output enables |
| `0x0420` | `JESD204B_CTRL` | — | `0x00` | JESD204B interface control |
| `0x0421` | `JESD204B_STATUS` | — | `0x00` | JESD204B link status |
| `0x0500` | `EEPROM_CTRL` | — | `0x00` | EEPROM control register |
| `0x0501` | `EEPROM_ADDR` | — | `0x0000` | EEPROM byte address |
| `0x0502` | `EEPROM_DATA` | — | `0x0000` | EEPROM read/write data |
| `0x0600` | `FLASH_CTRL` | — | `0x00` | Configuration Flash control (S25FL512S) |
| `0x0601` | `FLASH_ADDR_LOW` | — | `0x0000` | Flash address low word [15:0] |
| `0x0602` | `FLASH_ADDR_HIGH` | — | `0x00` | Flash address high word [23:16] |
| `0x0603` | `FLASH_DATA` | — | `0x0000` | Flash read/write data |
| `0x0604` | `FLASH_STATUS` | — | `0x01` | Flash operation status |
| `0x0700` | `RF_VGA_GAIN` | — | `0x20` | HMC698LP4 VGA gain control (6-bit SPI) |
| `0x0701` | `RF_CTRL` | — | `0x80` | RF front-end control |
| `0x0702` | `RF_STATUS` | — | `0x00` | RF front-end status |
| `0x0708` | `AGC_CONFIG` | — | `0x80` | Automatic Gain Control configuration |
| `0x0800` | `GPIO_DIR` | — | `0x00` | GPIO direction control |
| `0x0801` | `GPIO_DATA` | — | `0x00` | GPIO data write/read |
| `0x0802` | `GPIO_IN` | — | `0x00` | GPIO input read |
| `0x0803` | `GPIO_IRQ` | — | `0x00` | GPIO interrupt flags |
| `0x0900` | `SYSTEM_RESET` | — | `0x00` | System reset control |
| `0x0A00` | `I2C_CTRL` | — | `0x00` | I2C master control (LTC2975, LMK04828) |
| `0x0A01` | `I2C_STATUS` | — | `0x00` | I2C master status |
| `0x0A02` | `I2C_TXRX` | — | `0x00` | I2C transmit/receive data |

### `BOARD_ID` — Address `0x0000`

**Reset value:** `0x4B48`  **Access:** see fields below

Board identification code - kh Wideband RF Receiver unique identifier

| Field | Bits | Access | Reset | Description |
| --- | --- | --- | --- | --- |
| `BOARD_ID` | `[15:0]` | RO | `0x4B48` | Unique board identifier 0x4B48 (ASCII 'KH') |

### `BOARD_VERSION` — Address `0x0001`

**Reset value:** `0x01`  **Access:** see fields below

Hardware version number

| Field | Bits | Access | Reset | Description |
| --- | --- | --- | --- | --- |
| `MAJOR_VERSION` | `[7:4]` | RO | `0x0` | Major revision number |
| `MINOR_VERSION` | `[3:0]` | RO | `0x1` | Minor revision number |

### `BOARD_TYPE_ID` — Address `0x0002`

**Reset value:** `0x5246`  **Access:** see fields below

Board type identifier

| Field | Bits | Access | Reset | Description |
| --- | --- | --- | --- | --- |
| `TYPE_ID` | `[15:0]` | RO | `0x5246` | Board type: 0x5246 (ASCII 'RF') for RF Receiver |

### `SCRATCHPAD` — Address `0x0003`

**Reset value:** `0x0000`  **Access:** see fields below

Scratchpad register for RAM/CPU connectivity test

| Field | Bits | Access | Reset | Description |
| --- | --- | --- | --- | --- |
| `TEST_VALUE` | `[15:0]` | RW | `0x0000` | Read/write test pattern for RAM verification |

### `MCS_VERSION_MAJOR` — Address `0x0010`

**Reset value:** `0x01`  **Access:** see fields below

FPGA firmware major version number

| Field | Bits | Access | Reset | Description |
| --- | --- | --- | --- | --- |
| `MAJOR` | `[7:0]` | RO | `0x01` | Major firmware version |

### `MCS_VERSION_MINOR` — Address `0x0011`

**Reset value:** `0x00`  **Access:** see fields below

FPGA firmware minor version number

| Field | Bits | Access | Reset | Description |
| --- | --- | --- | --- | --- |
| `MINOR` | `[7:0]` | RO | `0x00` | Minor firmware version |

### `BUILD_DATE` — Address `0x0012`

**Reset value:** `0x2415`  **Access:** see fields below

FPGA build date in packed BCD format (YYYYMMDD)

| Field | Bits | Access | Reset | Description |
| --- | --- | --- | --- | --- |
| `YEAR` | `[15:12]` | RO | `0x2` | Build year (BCD) |
| `MONTH` | `[11:8]` | RO | `0x04` | Build month (BCD) |
| `DAY` | `[7:0]` | RO | `0x15` | Build day (BCD) |

### `UART_BAUD_DIV` — Address `0x0100`

**Reset value:** `0x0036`  **Access:** see fields below

UART baud rate divisor (115200 bps default)

| Field | Bits | Access | Reset | Description |
| --- | --- | --- | --- | --- |
| `DIVISOR` | `[15:0]` | RW | `0x0036` | Baud = CLK_FREQ / (16 * DIV), 54 = 115200@100MHz |

### `UART_CTRL` — Address `0x0101`

**Reset value:** `0x03`  **Access:** see fields below

UART control register

| Field | Bits | Access | Reset | Description |
| --- | --- | --- | --- | --- |
| `ENABLE` | `[0]` | RW | `0x0` | UART enable (1=enabled) |
| `LOOPBACK` | `[1]` | RW | `0x0` | Internal loopback for test |
| `PARITY_EN` | `[2]` | RW | `0x0` | Parity enable |
| `PARITY_EVEN` | `[3]` | RW | `0x0` | Parity type (1=even, 0=odd) |
| `FRAME_FORMAT` | `[7:4]` | RW | `0x3` | Frame format (bits, stop bits) |

### `UART_STATUS` — Address `0x0102`

**Reset value:** `0x00`  **Access:** see fields below

UART status flags (read-clear)

| Field | Bits | Access | Reset | Description |
| --- | --- | --- | --- | --- |
| `TX_BUSY` | `[0]` | RC | `0x0` | Transmitter busy flag |
| `RX_AVAIL` | `[1]` | RC | `0x0` | Data available in RX FIFO |
| `FRAME_ERR` | `[2]` | RC | `0x0` | Frame error detected |
| `PARITY_ERR` | `[3]` | RC | `0x0` | Parity error detected |

### `UART_TX_COUNT` — Address `0x0103`

**Reset value:** `0x00`  **Access:** see fields below

TX FIFO byte count

| Field | Bits | Access | Reset | Description |
| --- | --- | --- | --- | --- |
| `TX_COUNT` | `[7:0]` | RO | `0x00` | Number of bytes in TX FIFO |

### `UART_RX_COUNT` — Address `0x0104`

**Reset value:** `0x00`  **Access:** see fields below

RX FIFO byte count

| Field | Bits | Access | Reset | Description |
| --- | --- | --- | --- | --- |
| `RX_COUNT` | `[7:0]` | RO | `0x00` | Number of bytes in RX FIFO |

### `ADC_CTRL` — Address `0x0200`

**Reset value:** `0x00`  **Access:** see fields below

ADC control for power monitoring

| Field | Bits | Access | Reset | Description |
| --- | --- | --- | --- | --- |
| `START` | `[0]` | RW | `0x0` | Start ADC conversion |
| `CONTINUOUS` | `[1]` | RW | `0x0` | Continuous conversion mode |
| `CHANNEL_SEL` | `[3:2]` | RW | `0x0` | Input channel select |

### `ADC_STATUS` — Address `0x0201`

**Reset value:** `0x00`  **Access:** see fields below

ADC status flags

| Field | Bits | Access | Reset | Description |
| --- | --- | --- | --- | --- |
| `DATA_READY` | `[0]` | RC | `0x0` | New conversion data ready |
| `OVERRANGE` | `[1]` | RC | `0x0` | Input overrange detected |

### `VCC_5V_RAW` — Address `0x0210`

**Reset value:** `0x0000`  **Access:** see fields below

5V rail ADC raw count

| Field | Bits | Access | Reset | Description |
| --- | --- | --- | --- | --- |
| `ADC_COUNT` | `[11:0]` | RO | `0x000` | 12-bit ADC count, Volts = COUNT * 5.0/4096 |

### `VCC_3V3_RAW` — Address `0x0211`

**Reset value:** `0x0000`  **Access:** see fields below

3V rail ADC raw count

| Field | Bits | Access | Reset | Description |
| --- | --- | --- | --- | --- |
| `ADC_COUNT` | `[11:0]` | RO | `0x000` | 12-bit ADC count, Volts = COUNT * 3.3/4096 |

### `VCC_2V5_RAW` — Address `0x0212`

**Reset value:** `0x0000`  **Access:** see fields below

5V rail ADC raw count

| Field | Bits | Access | Reset | Description |
| --- | --- | --- | --- | --- |
| `ADC_COUNT` | `[11:0]` | RO | `0x000` | 12-bit ADC count (if present) |

### `VCC_1V8_RAW` — Address `0x0213`

**Reset value:** `0x0000`  **Access:** see fields below

8V rail ADC raw count

| Field | Bits | Access | Reset | Description |
| --- | --- | --- | --- | --- |
| `ADC_COUNT` | `[11:0]` | RO | `0x000` | 12-bit ADC count, Volts = COUNT * 1.8/4096 |

### `VCC_1V2_RAW` — Address `0x0214`

**Reset value:** `0x0000`  **Access:** see fields below

2V rail ADC raw count (FPGA Core)

| Field | Bits | Access | Reset | Description |
| --- | --- | --- | --- | --- |
| `ADC_COUNT` | `[11:0]` | RO | `0x000` | 12-bit ADC count, Volts = COUNT * 1.2/4096 |

### `ICC_5V_RAW` — Address `0x0218`

**Reset value:** `0x0000`  **Access:** see fields below

5V rail current sense ADC

| Field | Bits | Access | Reset | Description |
| --- | --- | --- | --- | --- |
| `ADC_COUNT` | `[11:0]` | RO | `0x000` | 12-bit ADC count for current |

### `ICC_3V3_RAW` — Address `0x0219`

**Reset value:** `0x0000`  **Access:** see fields below

3V rail current sense ADC

| Field | Bits | Access | Reset | Description |
| --- | --- | --- | --- | --- |
| `ADC_COUNT` | `[11:0]` | RO | `0x000` | 12-bit ADC count for current |

### `TEMP_LOCAL` — Address `0x0300`

**Reset value:** `0x0000`  **Access:** see fields below

Local FPGA die temperature

| Field | Bits | Access | Reset | Description |
| --- | --- | --- | --- | --- |
| `TEMP` | `[9:0]` | RO | `0x000` | Temperature in 0.25°C units (signed) |

### `TEMP_REMOTE1` — Address `0x0301`

**Reset value:** `0x0000`  **Access:** see fields below

Remote temperature sensor 1 (via LTC2975)

| Field | Bits | Access | Reset | Description |
| --- | --- | --- | --- | --- |
| `TEMP` | `[9:0]` | RO | `0x000` | Remote temperature in 0.25°C units (signed) |

### `TEMP_REMOTE2` — Address `0x0302`

**Reset value:** `0x0000`  **Access:** see fields below

Remote temperature sensor 2

| Field | Bits | Access | Reset | Description |
| --- | --- | --- | --- | --- |
| `TEMP` | `[9:0]` | RO | `0x000` | Remote temperature in 0.25°C units (signed) |

### `TEMP_ALERT_HIGH` — Address `0x0308`

**Reset value:** `0x0190`  **Access:** see fields below

Over-temperature alert threshold

| Field | Bits | Access | Reset | Description |
| --- | --- | --- | --- | --- |
| `THRESHOLD` | `[9:0]` | RW | `0x190` | Alert threshold in 0.25°C units |

### `TEMP_ALERT_LOW` — Address `0x0309`

**Reset value:** `0x0288`  **Access:** see fields below

Under-temperature alert threshold

| Field | Bits | Access | Reset | Description |
| --- | --- | --- | --- | --- |
| `THRESHOLD` | `[9:0]` | RW | `0x288` | Alert threshold (signed, 0x288 = -100°C) |

### `HEALTH_STATUS` — Address `0x030F`

**Reset value:** `0x00`  **Access:** see fields below

System health status flags

| Field | Bits | Access | Reset | Description |
| --- | --- | --- | --- | --- |
| `TEMP_OK` | `[0]` | RO | `0x0` | Temperature within limits (1=OK) |
| `VOLT_OK` | `[1]` | RO | `0x0` | All voltages within limits (1=OK) |
| `PLL_LOCK` | `[2]` | RO | `0x0` | PLL locked (1=LOCKED) |
| `JESD204B_SYNC` | `[3]` | RO | `0x0` | JESD204B link synchronized |
| `SYSTEM_OK` | `[7]` | RO | `0x0` | Overall system health (1=OK) |

### `PLL_CTRL` — Address `0x0400`

**Reset value:** `0x00`  **Access:** see fields below

PLL/LMK04828 control register

| Field | Bits | Access | Reset | Description |
| --- | --- | --- | --- | --- |
| `ENABLE` | `[0]` | RW | `0x0` | PLL enable (1=enabled) |
| `RESET` | `[1]` | RW | `0x0` | PLL reset (active high, self-clearing) |
| `REF_SEL` | `[3:2]` | RW | `0x0` | Reference clock select (00=10MHz osc) |

### `PLL_STATUS` — Address `0x0401`

**Reset value:** `0x00`  **Access:** see fields below

PLL status flags

| Field | Bits | Access | Reset | Description |
| --- | --- | --- | --- | --- |
| `LOCKED` | `[0]` | RO | `0x0` | PLL locked indicator |
| `LOSS_OF_LOCK` | `[1]` | RC | `0x0` | PLL loss of lock event |

### `PLL_N_DIV` — Address `0x0402`

**Reset value:** `0x0064`  **Access:** see fields below

PLL N divider value

| Field | Bits | Access | Reset | Description |
| --- | --- | --- | --- | --- |
| `N_DIVIDER` | `[15:0]` | RW | `0x0064` | Integer N divider value |

### `PLL_R_DIV` — Address `0x0403`

**Reset value:** `0x01`  **Access:** see fields below

PLL R divider value

| Field | Bits | Access | Reset | Description |
| --- | --- | --- | --- | --- |
| `R_DIVIDER` | `[7:0]` | RW | `0x01` | R divider value |

### `CLK_ENABLE` — Address `0x0410`

**Reset value:** `0x00`  **Access:** see fields below

Clock output enables

| Field | Bits | Access | Reset | Description |
| --- | --- | --- | --- | --- |
| `CLK_ADC_EN` | `[0]` | RW | `0x0` | ADC sample clock enable |
| `CLK_FPGA_EN` | `[1]` | RW | `0x0` | FPGA fabric clock enable |
| `CLK_JESD_EN` | `[2]` | RW | `0x0` | JESD204B lane clock enable |

### `JESD204B_CTRL` — Address `0x0420`

**Reset value:** `0x00`  **Access:** see fields below

JESD204B interface control

| Field | Bits | Access | Reset | Description |
| --- | --- | --- | --- | --- |
| `ENABLE` | `[0]` | RW | `0x0` | JESD204B link enable |
| `RESET` | `[1]` | RW | `0x0` | JESD204B reset (self-clearing) |
| `LANE_MODE` | `[3:2]` | RW | `0x0` | Lane mode (00=dual, 01=interleaved) |

### `JESD204B_STATUS` — Address `0x0421`

**Reset value:** `0x00`  **Access:** see fields below

JESD204B link status

| Field | Bits | Access | Reset | Description |
| --- | --- | --- | --- | --- |
| `SYNC` | `[0]` | RO | `0x0` | Link synchronized (SYNC~ asserted) |
| `ALIGN_DONE` | `[1]` | RO | `0x0` | Lane alignment complete |

### `EEPROM_CTRL` — Address `0x0500`

**Reset value:** `0x00`  **Access:** see fields below

EEPROM control register

| Field | Bits | Access | Reset | Description |
| --- | --- | --- | --- | --- |
| `READ` | `[0]` | RW | `0x0` | Initiate read (self-clearing) |
| `WRITE` | `[1]` | RW | `0x0` | Initiate write (self-clearing) |
| `ERASE` | `[2]` | RW | `0x0` | Initiate erase (self-clearing) |
| `BUSY` | `[7]` | RO | `0x0` | EEPROM busy flag |

### `EEPROM_ADDR` — Address `0x0501`

**Reset value:** `0x0000`  **Access:** see fields below

EEPROM byte address

| Field | Bits | Access | Reset | Description |
| --- | --- | --- | --- | --- |
| `ADDRESS` | `[15:0]` | RW | `0x0000` | 16-bit byte address |

### `EEPROM_DATA` — Address `0x0502`

**Reset value:** `0x0000`  **Access:** see fields below

EEPROM read/write data

| Field | Bits | Access | Reset | Description |
| --- | --- | --- | --- | --- |
| `DATA` | `[15:0]` | RW | `0x0000` | 16-bit data word |

### `FLASH_CTRL` — Address `0x0600`

**Reset value:** `0x00`  **Access:** see fields below

Configuration Flash control (S25FL512S)

| Field | Bits | Access | Reset | Description |
| --- | --- | --- | --- | --- |
| `READ` | `[0]` | RW | `0x0` | Initiate read (self-clearing) |
| `WRITE` | `[1]` | RW | `0x0` | Initiate program (self-clearing) |
| `ERASE_SECTOR` | `[2]` | RW | `0x0` | Erase sector (self-clearing) |
| `ERASE_CHIP` | `[3]` | RW | `0x0` | Erase entire chip (self-clearing) |
| `BUSY` | `[7]` | RO | `0x0` | Flash busy flag |

### `FLASH_ADDR_LOW` — Address `0x0601`

**Reset value:** `0x0000`  **Access:** see fields below

Flash address low word [15:0]

| Field | Bits | Access | Reset | Description |
| --- | --- | --- | --- | --- |
| `ADDR_LOW` | `[15:0]` | RW | `0x0000` | Flash address bits [15:0] |

### `FLASH_ADDR_HIGH` — Address `0x0602`

**Reset value:** `0x00`  **Access:** see fields below

Flash address high word [23:16]

| Field | Bits | Access | Reset | Description |
| --- | --- | --- | --- | --- |
| `ADDR_HIGH` | `[7:0]` | RW | `0x00` | Flash address bits [23:16] |

### `FLASH_DATA` — Address `0x0603`

**Reset value:** `0x0000`  **Access:** see fields below

Flash read/write data

| Field | Bits | Access | Reset | Description |
| --- | --- | --- | --- | --- |
| `DATA` | `[15:0]` | RW | `0x0000` | 16-bit data word |

### `FLASH_STATUS` — Address `0x0604`

**Reset value:** `0x01`  **Access:** see fields below

Flash operation status

| Field | Bits | Access | Reset | Description |
| --- | --- | --- | --- | --- |
| `READY` | `[0]` | RO | `0x1` | Flash ready for operations |
| `WRITE_ERR` | `[1]` | RC | `0x0` | Write/program error |
| `ERASE_ERR` | `[2]` | RC | `0x0` | Erase error |

### `RF_VGA_GAIN` — Address `0x0700`

**Reset value:** `0x20`  **Access:** see fields below

HMC698LP4 VGA gain control (6-bit SPI)

| Field | Bits | Access | Reset | Description |
| --- | --- | --- | --- | --- |
| `GAIN_CODE` | `[5:0]` | RW | `0x20` | 6-bit gain code (0-63, 31.5dB range, 0.5dB/step) |

### `RF_CTRL` — Address `0x0701`

**Reset value:** `0x80`  **Access:** see fields below

RF front-end control

| Field | Bits | Access | Reset | Description |
| --- | --- | --- | --- | --- |
| `LNA_ENABLE` | `[0]` | RW | `0x0` | LNA enable (HMC1119LP4DE) |
| `VGA_ENABLE` | `[1]` | RW | `0x0` | VGA enable (HMC698LP4) |
| `MIXER_ENABLE` | `[2]` | RW | `0x0` | Mixer enable (HMC1051LP4BE) |
| `RF_POWER_DOWN` | `[7]` | RW | `0x1` | RF chain power-down (1=PD) |

### `RF_STATUS` — Address `0x0702`

**Reset value:** `0x00`  **Access:** see fields below

RF front-end status

| Field | Bits | Access | Reset | Description |
| --- | --- | --- | --- | --- |
| `LNA_OK` | `[0]` | RO | `0x0` | LNA powered/operational |
| `VGA_OK` | `[1]` | RO | `0x0` | VGA powered/operational |
| `MIXER_OK` | `[2]` | RO | `0x0` | Mixer powered/operational |

### `AGC_CONFIG` — Address `0x0708`

**Reset value:** `0x80`  **Access:** see fields below

Automatic Gain Control configuration

| Field | Bits | Access | Reset | Description |
| --- | --- | --- | --- | --- |
| `ENABLE` | `[0]` | RW | `0x0` | AGC enable |
| `TARGET_LEVEL` | `[7:4]` | RW | `0x8` | Target ADC level (4-bit) |

### `GPIO_DIR` — Address `0x0800`

**Reset value:** `0x00`  **Access:** see fields below

GPIO direction control

| Field | Bits | Access | Reset | Description |
| --- | --- | --- | --- | --- |
| `GPIO_DIR` | `[7:0]` | RW | `0x00` | 0=input, 1=output |

### `GPIO_DATA` — Address `0x0801`

**Reset value:** `0x00`  **Access:** see fields below

GPIO data write/read

| Field | Bits | Access | Reset | Description |
| --- | --- | --- | --- | --- |
| `GPIO_OUT` | `[7:0]` | RW | `0x00` | GPIO output data |

### `GPIO_IN` — Address `0x0802`

**Reset value:** `0x00`  **Access:** see fields below

GPIO input read

| Field | Bits | Access | Reset | Description |
| --- | --- | --- | --- | --- |
| `GPIO_IN` | `[7:0]` | RO | `0x00` | GPIO input data |

### `GPIO_IRQ` — Address `0x0803`

**Reset value:** `0x00`  **Access:** see fields below

GPIO interrupt flags

| Field | Bits | Access | Reset | Description |
| --- | --- | --- | --- | --- |
| `IRQ_FLAGS` | `[7:0]` | RC | `0x00` | Interrupt flags (1=pending) |

### `SYSTEM_RESET` — Address `0x0900`

**Reset value:** `0x00`  **Access:** see fields below

System reset control

| Field | Bits | Access | Reset | Description |
| --- | --- | --- | --- | --- |
| `RESET_FPGA` | `[0]` | RW | `0x0` | FPGA logic reset (self-clearing) |
| `RESET_RF` | `[1]` | RW | `0x0` | RF chain reset |
| `RESET_ADC` | `[2]` | RW | `0x0` | ADC interface reset |
| `GLOBAL_RESET` | `[7]` | RW | `0x0` | Global system reset (self-clearing) |

### `I2C_CTRL` — Address `0x0A00`

**Reset value:** `0x00`  **Access:** see fields below

I2C master control (LTC2975, LMK04828)

| Field | Bits | Access | Reset | Description |
| --- | --- | --- | --- | --- |
| `ENABLE` | `[0]` | RW | `0x0` | I2C master enable |
| `START` | `[1]` | RW | `0x0` | Generate START condition |
| `STOP` | `[2]` | RW | `0x0` | Generate STOP condition |
| `RW_BIT` | `[3]` | RW | `0x0` | R/W bit (0=write, 1=read) |

### `I2C_STATUS` — Address `0x0A01`

**Reset value:** `0x00`  **Access:** see fields below

I2C master status

| Field | Bits | Access | Reset | Description |
| --- | --- | --- | --- | --- |
| `BUSY` | `[0]` | RO | `0x0` | I2C bus busy |
| `ACK` | `[1]` | RO | `0x0` | Received ACK (1=ACK) |
| `ARB_LOST` | `[2]` | RC | `0x0` | Arbitration lost |

### `I2C_TXRX` — Address `0x0A02`

**Reset value:** `0x00`  **Access:** see fields below

I2C transmit/receive data

| Field | Bits | Access | Reset | Description |
| --- | --- | --- | --- | --- |
| `DATA` | `[7:0]` | RW | `0x00` | Transmit or received byte |