# Hardware Requirements

## khgk

## 1. Project Summary

Wideband RF receiver module covering 5-18 GHz frequency range with 13 GHz instantaneous bandwidth, military temperature operation (-55 to +125°C), and JESD204B/C high-speed digital output interface for radar and electronic warfare applications.

## 2. Design Parameters

| Parameter | Value |
| --- | --- |
| Rf Frequency Range | 5-18 GHz |
| Instantaneous Bandwidth | 13 GHz |
| Noise Figure Target | 3-6 dB (system) |
| Sfdr Target | 60-80 dB |
| Adc Sample Rate | 1-2 GSPS |
| Adc Resolution | 12-14 bits |
| Digital Interface | JESD204B/C, up to 4 lanes, 6-12.5 Gbps per lane |
| Power Consumption | 10-30W (target 15-20W) |
| Supply Voltage | 12V or 15V DC nominal |
| Operating Temperature | -55 to +125°C (military) |
| Phase Noise | -80 to -90 dBc/Hz @ 1-10 kHz offset |
| Input P1Db | -20 to 0 dBm (system) |
| Input Ip3 | +10 to +20 dBm (system) |
| Gain Range | 0-60 dB programmable |
| Tuning Speed | <5 µs |
| Form Factor | 120mm x 80mm x 15mm maximum |
| Input Connector | SMA or SMP |
| Control Interface | SPI/I2C |
| Compliance Standards | ['MIL-STD-810G', 'MIL-STD-461G', 'RoHS'] |

## 3. Requirements

### 3.1 Functional Requirements

| ID | Title | Description | Priority | Validation | Dependencies | Constraints |
| --- | --- | --- | --- | --- | --- | --- |
| REQ-HW-001 | RF Input Frequency Range | Receiver shall accept RF input signals from 5.0 GHz to 18.0 GHz with a minimum of 13 GHz instantaneous bandwidth coverage. | Must have | test | None | Wideband input matching required across entire band, Input return loss ≤ -10 dB required |
| REQ-HW-004 | Gain Control Range | Receiver shall provide programmable gain adjustment from 0 dB to at least 60 dB of total system gain to accommodate varying input signal levels. | Must have | test | None | Step size: 1 dB or finer, Settling time < 1 µs |
| REQ-HW-014 | Frequency Agility and Tuning Speed | Receiver shall support fast frequency hopping with tuning time not exceeding 5 µs for full band frequency changes. | Should have | test | REQ-HW-007 | Synthesizer lock time < 5 µs |

### 3.2 Performance Requirements

| ID | Title | Description | Priority | Validation | Dependencies | Constraints |
| --- | --- | --- | --- | --- | --- | --- |
| REQ-HW-002 | Noise Figure Performance | System noise figure shall not exceed 6.0 dB across the 5-18 GHz operating band, with target NF of 3-4 dB in optimal frequency ranges. | Must have | test | REQ-HW-004 | LNA gain sufficient to overcome mixer conversion loss |
| REQ-HW-003 | Spurious-Free Dynamic Range | Receiver shall provide minimum 60 dB SFDR with target of 70-80 dB SFDR across all frequency bands and temperature ranges. | Must have | test | REQ-HW-007, REQ-HW-008 | IP3 and P1dB must support SFDR requirement |
| REQ-HW-006 | ADC Sampling Rate | System shall digitize signal at minimum 1.0 GSPS with supporting bandwidth of 500 MHz or greater per complex channel. | Must have | test | None | ADC resolution: 12-14 bits minimum, ENOB > 9 bits at Nyquist |
| REQ-HW-007 | Phase Noise Performance | Local oscillator phase noise shall not exceed -80 dBc/Hz at 1 kHz offset and -90 dBc/Hz at 10 kHz offset from carrier across 5-18 GHz tuning range. | Must have | test | None | Integrated phase noise < 1 degree RMS |
| REQ-HW-008 | Linearity and Intercept Points | Receiver cascade shall achieve minimum input IP3 of +10 dBm and output P1dB of not less than 0 dBm to meet 60 dB SFDR requirement. | Must have | test | None | Gain distribution must optimize IP3 |
| REQ-HW-013 | Input Return Loss | RF input shall maintain return loss of 10 dB or better (VSWR ≤ 2:1) across the 5-18 GHz operating band. | Must have | test | REQ-HW-001 | Input matching network must maintain performance over temperature |

### 3.3 Interface Requirements

| ID | Title | Description | Priority | Validation | Dependencies | Constraints |
| --- | --- | --- | --- | --- | --- | --- |
| REQ-HW-005 | JESD204B/C Output Interface | Digitized IF/baseband samples shall be output via JESD204B or JESD204C high-speed serial interface supporting data rates up to 12.5 Gbps per lane. | Must have | test | REQ-HW-006 | Support for subclass 1 deterministic latency, Maximum lane count: 4 lanes, Lane data rate: 6-12.5 Gbps |
| REQ-HW-012 | Control Interface | Receiver shall provide SPI or I2C control interface for gain setting, frequency tuning, and configuration registers. | Must have | test | None | Maximum SPI clock: 10 MHz, Support for multi-byte register addressing |

### 3.4 Environmental Requirements

| ID | Title | Description | Priority | Validation | Dependencies | Constraints |
| --- | --- | --- | --- | --- | --- | --- |
| REQ-HW-009 | Operating Temperature Range | Receiver shall maintain full performance specifications across -55°C to +125°C ambient temperature range (military temperature grade). | Must have | test | None | All components shall be MIL-PRF-38535 or equivalent, Temperature compensation required for gain and frequency drift |
| REQ-HW-015 | Vibration and Shock Requirements | Design shall withstand military environmental conditions per MIL-STD-810G for tactical applications (vibration: 5-2000 Hz, 20g shock). | Should have | test | None | PCB stiffening required, Component mounting with appropriate standoffs |

### 3.5 Constraint Requirements

| ID | Title | Description | Priority | Validation | Dependencies | Constraints |
| --- | --- | --- | --- | --- | --- | --- |
| REQ-HW-010 | Power Consumption Budget | Total receiver power consumption shall not exceed 30W from nominal +12V or +15V DC supply, with target consumption of 15-20W typical. | Must have | test | None | Power sequencing required for multiple voltage rails, Overcurrent protection required |
| REQ-HW-011 | Form Factor and Mechanical Constraints | Receiver shall be implemented as compact module suitable for tactical deployment with maximum dimensions not exceeding 120mm x 80mm x 15mm (L x W x H). | Should have | inspection | None | SMA or SMP RF input connectors, High-density military-grade circular connector for digital/control I/O, Conformal coating required for environmental protection |
| REQ-HW-016 | EMI/EMC Compliance | Receiver shall meet MIL-STD-461G requirements for conducted and radiated emissions (RE102, CE102) and susceptibility (RS103, CS101). | Should have | test | None | EMI gaskets required on enclosure seams, Proper filtering on all DC and control lines |
| REQ-HW-017 | Component Sourcing and Lifecycle | All components shall be selected from manufacturers providing long-term availability (minimum 10-year lifecycle forecast) with military grade or equivalent screening options. | Should have | inspection | None | Avoid sole-source components where possible, Prefer QML-qualified devices for critical functions |