# Glue Logic Requirements (GLR)

## Document Control

| Document Title | Glue Logic Requirements |
| --- | --- |
| Version Date | 16.04.2026 |
| Version Number | 0V01 |
| Prepared By | Name: . Sign: |
| Document Review By | Name: . Sign: |

## Amendments to the Document

| S. No. | Ver. No. | Ver. Date | Changed By | Section(s) Changed | Description of Change |
| --- | --- | --- | --- | --- | --- |
| 1 | 0V01 | 16.04.2026 | - | - | Initial Version |

## 1. Scope of the Document

This document explains the IO details and functional requirements of the FPGA for **dkfjg**. It bridges the hardware netlist (P4) and the FPGA HDL design (P7), defining the signal interfaces, power sequencing, control protocols, and register map required to operate the 5–18 GHz RF receiver chain.

Targeted audience: Hardware Design, Firmware, and Systems Engineering teams.

## 2. References

### 2.1 External

| Doc. Type | Part No. | Description |
| --- | --- | --- |
| Datasheet | HMC1042LP4BE | Wideband LNA, 6-20 GHz |
| Datasheet | HMC698LP4 | Digital Variable Gain Amplifier, 5-20 GHz |
| Datasheet | HMC774A | Double-Balanced Mixer, 6-20 GHz |
| Datasheet | ADA4817 | Low Noise, Fast settling Op-Amp (IF Amp) |
| Datasheet | ADC12J4000 | 12-Bit, 4 GSPS ADC, JESD204B |
| Datasheet | LMK04828 | Ultra-Low Noise JESD204B Clock Generator |
| Datasheet | ADF4355 | Wideband Synthesizer with Integrated VCO |
| Datasheet | LTC2992 | Power/Energy Monitor |
| Datasheet | LTM4644 | Quad 4A DC-DC Buck Regulator |
| Datasheet | Kintex-7 | FPGA Family Datasheet (Device: XC7K70T) |

### 2.2 Internal

| Reference | Document |
| --- | --- |
| [HRS] | Hardware Requirements Specification (dkfjg) |
| [SCH] | System Schematic |
| [GRS] | General Requirements Specification |
| [GDD] | General Design Document |

## 3. Acronyms and Abbreviations

| Acronym | Expansion |
| --- | --- |
| ADC | Analog-to-Digital Converter |
| BGA | Ball Grid Array |
| CDR | Clock and Data Recovery |
| CLB | Configurable Logic Block |
| DAC | Digital-to-Analog Converter |
| DMA | Direct Memory Access |
| DSP | Digital Signal Processing |
| EMC | Electromagnetic Compatibility |
| FF | Flip-Flop |
| FIFO | First-In-First-Out |
| FPGA | Field-Programmable Gate Array |
| GND | Ground Reference |
| GPIO | General Purpose Input/Output |
| HDL | Hardware Description Language |
| I2C | Inter-Integrated Circuit |
| IIP3 | Input Third-Order Intercept Point |
| JESD | JESD204 Standard (High-speed ADC interface) |
| JTAG | Joint Test Action Group |
| LDO | Low Dropout Regulator |
| LO | Local Oscillator |
| LUT | Look-Up Table |
| LVDS | Low-Voltage Differential Signaling |
| LVTTL | Low Voltage TTL |
| NF | Noise Figure |
| PCB | Printed Circuit Board |
| PLL | Phase-Locked Loop |
| RAM | Random Access Memory |
| RF | Radio Frequency |
| ROM | Read-Only Memory |
| RTL | Register Transfer Level |
| RX | Receive |
| SNR | Signal-to-Noise Ratio |
| SPI | Serial Peripheral Interface |
| UART | Universal Asynchronous Receiver-Transmitter |
| VHDL | VHSIC Hardware Description Language |
| VCC | Voltage Supply |

## 4. Module Overview

**RF SECTION:**

The RF Front-end consists of a wideband LNA (HMC1042LP4BE) providing 24 dB gain, followed by a digitally controlled VGA (HMC698LP4) offering 0-22 dB gain adjustment in 1 dB steps. The signal passes through a Bandpass Filter (CBP-1810-LF) and is downconverted by a Mixer (HMC774A) using an LO signal generated by ADF4355. The IF signal is amplified by ADA4817 before digitization. This chain covers 5-18 GHz with high linearity (IIP3 >20 dBm).

**DIGITAL SECTION:**

The digital core is based on a Xilinx Kintex-7 FPGA (XC7K70T). The FPGA acts as the system controller, configuring the RF path via SPI/I2C (VGA gain, PLL frequency), monitoring system health via I2C (LTC2992), and managing power sequencing. It receives digitized IF data via JESD204B from the ADC12J4000 and processes it for transmission. It also handles the user interface command protocol (UART) and system status LEDs.

**POWER SUPPLY SECTION:**

Power is derived from a 12V DC input (J4). The LTM4644 DC-DC buck converter generates intermediate rails. LT3045 (5V) supplies RF components (LNA, Mixer, PLL). LT3042 (3.3V) supplies FPGA I/O and auxiliary logic. LT3033 (1.8V) supplies FPGA VCCAUX. The FPGA core voltage (1.0V) is generated by an integrated regulator on the LTM4644 or a dedicated rail.

## 5. Features

**FPGA:** Xilinx XC7K70T-484FBG484C (Kintex-7 Family).

**On-board clock:** 125 MHz LVDS Oscillator for FPGA system clock.

**High-Speed Data Link:** JESD204B Subclass 1 (8 lanes) interface to ADC12J4000 @ 4 GSPS.

**Control Interfaces:**

SPI Master for VGA (HMC698LP4), Synthesizer (ADF4355), and Flash.

I2C Master for Clock Gen (LMK04828) and Power Monitor (LTC2992).

**Debug Interface:** JTAG (14-pin) for FPGA programming and debug.

**RF Control:** 3-wire Serial Interface for VGA Gain control; SPI for PLL configuration.

**Power Management:** Real-time monitoring of Voltage, Current, and Temperature via LTC2992.

**External Comms:** UART (RS-232/422 transceiver compatible) for host control.

**Storage:** SPI Flash (128 Mb) for FPGA configuration and Non-volatile parameter storage.

## 6. FPGA Description

The **Xilinx Kintex-7 XC7K70T** is selected for its high-performance DSP slices (necessary for future signal processing expansion) and dedicated Gigabit Transceivers (GTX) supporting the JESD204B interface required by the 4 GSPS ADC. The 484-pin BGA package offers sufficient I/O density for the parallel control of the RF front end.

| S.NO | PARAMETERS | SPECIFICATION |
| --- | --- | --- |
| 1 | Part Number | XC7K70T-484FBG484C |
| 2 | Logic Cells | 55,280 |
| 3 | CLB Flip-Flops | 69,120 |
| 4 | Number of Gates | 1.1 Million |
| 5 | Maximum Distributed RAM (Kb) | 375 |
| 6 | Total Block RAM (Kb) | 270 |
| 7 | Maximum Single-Ended I/Os | 300 |
| 8 | Maximum DSP Slices | 240 |
| 9 | No of IO Bank | 14 (3.3V capable) |
| 10 | GTX Transceivers | 4 (Supporting JESD204B) |

## 7. Block Diagram

*(Refer to System Block Diagram in Section 4 of this document. The logic within the FPGA is divided into: System Control (UART Arbiter), Configuration Managers (SPI/I2C Masters), JESD204B IP Core, and Monitoring Logic.)*

## 8. Pinout Details

**Table: FPGA Pin Out Details**

| S.No | Signal Name | Pin No | Voltage Level | Direction wrt FPGA | Source | Destination | Default Condition | Voltage Standard |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | VCCINT | - | 1.0V | PWR | LTM4644_CH4 | FPGA Core | - | - |
| 2 | VCCAUX | - | 1.8V | PWR | LT3033 | FPGA Aux | - | - |
| 3 | VCCO_34 | - | 3.3V | PWR | LT3042 | Bank 34 | - | - |
| 4 | GND | - | 0V | PWR | GND Plane | FPGA GND | - | - |
| 5 | FPGA_CLK_125M_P | AB12 | 1.8V | IN | Oscillator | PLL/MMCM | Oscillating | LVDS |
| 6 | FPGA_CLK_125M_N | AC12 | 1.8V | IN | Oscillator | PLL/MMCM | Oscillating | LVDS |
| 7 | JTAG_TCK | T13 | 1.8V | IN | Connector | JTAG TAP | Weak Pull-up | LVCMOS18 |
| 8 | JTAG_TDI | R13 | 1.8V | IN | Connector | JTAG TAP | Weak Pull-up | LVCMOS18 |
| 9 | JTAG_TDO | P14 | 1.8V | OUT | JTAG TAP | Connector | High Z | LVCMOS18 |
| 10 | JTAG_TMS | N13 | 1.8V | IN | Connector | JTAG TAP | Weak Pull-up | LVCMOS18 |
| 11 | RESET_N | M15 | 3.3V | IN | System Reset | Sys Reset | Pull-up | LVCMOS33 |
| 12 | UART_TX | K16 | 3.3V | OUT | FPGA IP | Host/Transceiver | High | LVCMOS33 |
| 13 | UART_RX | L16 | 3.3V | IN | Host/Transceiver | FPGA IP | High Z | LVCMOS33 |
| 14 | SPI_SCLK | A12 | 3.3V | OUT | FPGA SPI Master | VGA/Flash/PLL | Low | LVCMOS33 |
| 15 | SPI_MOSI | B12 | 3.3V | OUT | FPGA SPI Master | VGA/Flash/PLL | Low | LVCMOS33 |
| 16 | SPI_MISO | C12 | 3.3V | IN | VGA/Flash/PLL | FPGA SPI Master | High Z | LVCMOS33 |
| 17 | VGA_CS_N | D12 | 3.3V | OUT | FPGA SPI Master | VGA (HMC698LP4) | High | LVCMOS33 |
| 18 | PLL_CS_N | E12 | 3.3V | OUT | FPGA SPI Master | LO Synth (ADF4355) | High | LVCMOS33 |
| 19 | FLASH_CS_N | F12 | 3.3V | OUT | FPGA SPI Master | Config Flash | High | LVCMOS33 |
| 20 | I2C_SCL | G12 | 3.3V | Bi-Dir | FPGA I2C Master | Temp/Clock/Pwr | High (Pull-up) | LVCMOS33 |
| 21 | I2C_SDA | H12 | 3.3V | Bi-Dir | FPGA I2C Master | Temp/Clock/Pwr | High (Pull-up) | LVCMOS33 |
| 22 | VGA_LE | J12 | 3.3V | OUT | FPGA Control | VGA Latch | Low | LVCMOS33 |
| 23 | VGA_CLK | K12 | 3.3V | OUT | FPGA Control | VGA Serial Clk | Low | LVCMOS33 |
| 24 | VGA_DATA | L12 | 3.3V | OUT | FPGA Control | VGA Data In | Low | LVCMOS33 |
| 25 | ADC_JESD_TXP_0 | R1 | 1.0V | OUT | FPGA GTX | ADC JESD Rx | CDR | CML_1.0V |
| 26 | ADC_JESD_TXN_0 | T1 | 1.0V | OUT | FPGA GTX | ADC JESD Rx | CDR | CML_1.0V |
| 27 | ADC_JESD_RXP_0 | V1 | 1.0V | IN | ADC JESD Tx | FPGA GTX | AC Coupled | CML_1.0V |
| 28 | ADC_JESD_RXN_0 | W1 | 1.0V | IN | ADC JESD Tx | FPGA GTX | AC Coupled | CML_1.0V |
| 29 | ADC_SYNC_P | Y1 | 1.0V | OUT | FPGA GTX | ADC SYNC | High | LVDS |
| 30 | ADC_SYNC_N | AA1 | 1.0V | OUT | FPGA GTX | ADC SYNC | High | LVDS |
| 31 | PWR_GOOD | M14 | 3.3V | IN | PMIC (LTM4644) | FPGA Mon | High | LVCMOS33 |
| 32 | TRP | N14 | 3.3V | OUT | FPGA Control | RF Front-End Ctrl | Low | LVCMOS33 |
| 33 | LED_STATUS | P15 | 3.3V | OUT | FPGA Core | LED (Green) | Low | LVCMOS33 |
| 34 | LED_ERROR | R16 | 3.3V | OUT | FPGA Core | LED (Red) | Low | LVCMOS33 |
| 35 | TEMP_ALERT | T15 | 3.3V | IN | Thermal Sensor | FPGA Interrupt | Low | LVCMOS33 |
| 36 | CLK_REF_OUT | U16 | 3.3V | OUT | FPGA Clock | Ext/Future Use | Low | LVCMOS33 |
| 37 | FPGA_DONE | - | 1.8V | OUT | FPGA Config | Indicator | Low (Init) | - |

*(Note: JESD204B uses 4 lanes in the Netlist (P4), mapped to GTX channels. Pin numbers are illustrative placements for Bank 114 (GTX) and general I/O banks.)*

## 9. Functional Specifications

**Summary table:**

| S.No. | Function Name | Description |
| --- | --- | --- |
| 1 | Serial Communication Interface | UART between PC & FPGA for setup/control (via Header J5) |
| 2 | High Speed Communication Interface | JESD204B Subclass 1 (4 Lanes, 4.0 Gbps) to ADC12J4000 |
| 3 | Power Supply Sequencing & Health Status | Controls enable pins for regulators and monitors PWR_GOOD |
| 4 | Supply Voltage, Current & Temperature Monitoring | I2C-based monitoring via LTC2992 |
| 5 | Flash Interfaces | SPI Configuration Flash and Storage |
| 6 | TRP Configuration | Transmit/Receive Pulse or general RF Enable control |
| 7 | FPGA Remote Programming | Configuration loading via UART (Legacy) |
| 8 | VGA Controlling | Digital Attenuation control via 3-wire Serial and SPI |
| 9 | Beam/Frequency Tuning | ADF4355 PLL frequency setting via SPI |

### 9.1 Serial Communication Interface

**Interface type:** UART

**Physical layer:** TTL (Level shifted to RS-232/422 by Host)

**Baud rate:** 3,000,000 bps (3 Mbps default)

**Frame format:** 1 start bit, 8 data bits, 1 stop bit, no parity (8N1)

**Connector:** Header J5 (Control Interface)

**Signals:** UART_TX (FPGA → PC), UART_RX (PC → FPGA)

**Protocol:** Custom register-based command/response defined in Section 11.

### 9.2 High Speed Communication Interface

**Interface:** JESD204B Subclass 1

**Number of lanes:** 4 (bonded mode)

**Data rate per lane:** 4.0 Gbps (Configurable)

**Protocol:** JESD204B

**Physical:** AC-coupled LVDS connections to ADC12J4000.

**Sync:** Deterministic latency using SYSREF from LMK04828.

### 9.3 Power On/Off Sequence

#### 9.3.1 Power ON/OFF Sequence

**Input Supply (+12V) applied:** LTM4644 (U10) enables.

**Rail Ramp-up:** 5V, 3.3V, 1.8V, 1.0V ramp up per PMIC requirements.

**Power Good Assertion:** PMIC asserts PWR_GOOD.

**FPGA Release:** FPGA finishes configuration (DONE = High).

**RF Enable:** FPGA waits for stability, then asserts TRP (RF Enable) and enables VGA EN pin.

**Normal Operation:** ADC Data capture begins.

#### 9.3.2 Mode Configuration

| Mode | Signal | Value | Description |
| --- | --- | --- | --- |
| Normal | MODE[1:0] | 2'b00 | Default operating mode |
| Calib | MODE[1:0] | 2'b01 | Internal calibration loop |
| Programming | MODE[1:0] | 2'b11 | Flash programming mode via UART |

### 9.4 Supply Voltage, Current & Temperature Monitoring

#### 9.4.1 Supply Voltage and Current Monitoring

**IC Part Number:** LTC2992

**Interface:** I2C (Address 0x6C)

**Monitored Rails:** 12V Input, 5V RF rail, 3.3V Digital rail, 1.0V Core rail.

**Measurement range:** 0 to 60V, 0 to 10A (via sense resistors).

**Resolution:** 16-bit Delta-Sigma ADC.

#### 9.4.2 Temperature Monitoring

**IC Part Number:** LTC2992 (Internal) + ADC12J4000 Internal.

**Interface:** I2C (LTC2992), JESD204B (ADC internal temp via bits).

**Temperature range:** -55°C to +125°C.

**Alert threshold:** 120°C (Hardware alarm re-triggers reset if exceeded).

### 9.5 Flash & Interfaces

#### 9.5.1 Configuration Flash

**Part Number:** S25FL128S (128 Mb)

**Interface:** SPI (x4 Mode supported)

**Purpose:** Stores FPGA bitstream and User calibration data.

**Programming:** Internal FPGA Master or JTAG.

#### 9.5.2 Storage Flash (User)

Same physical device, partitioned. Stores LUTs for Gain/Phase correction.

### 9.6 TRP Configuration

**Signal:** TRP (General RF Enable)

**Direction:** FPGA → RF Front-end (via J5 or dedicated trace).

**Logic level:** 3.3V LVTTL

**Active state:** HIGH = RF Chain Powered/Enabled.

**Control:** Via Register `RF_CTRL` (Offset 0x04) in Register Map.

### 9.7 FPGA Remote Programming

**Protocol:** UART (defined in Section 11).

**Procedure:**

Host sends "Enter Prog Mode" command (Special Frame).

FPGA erases Flash sector.

Bitstream transferred in 4KB packets.

CRC verification.

FPGA Soft-Reset and reload.

### 9.8 VGA Controlling

**Interface:** 3-Wire Serial (CLK, DATA, LE) & SPI Fallback.

**Component:** HMC698LP4.

**Operation:**

**3-Wire Mode:** FPGA toggles `VGA_CLK`, shifts data on `VGA_DATA`, pulses `VGA_LE` to latch. Used for fast gain changes (6-bit gain code).

**SPI Mode:** Full configuration access via `SPI_CS_N` and `SPI_MOSI`.

### 9.9 Frequency Tuning (PLL)

**Component:** ADF4355.

**Interface:** SPI (Mode 0/1 compatible).

**Operation:**

FPGA writes 4 registers (Int, Frac, Mod, R) to ADF4355.

Update bit toggles to calibrate VCO.

Frequency range 5-18 GHz supported via fundamental VCO or dividers.

## 10. Software Register Address Map

### 10.1 Register Base Addresses

| Block Name | Base Address | Address Range | Description |
| --- | --- | --- | --- |
| System / Identification | 0x0000 | 0x0000–0x00FF | Board ID, firmware version, status |
| UART Control | 0x0100 | 0x0100–0x01FF | Baud rate, FIFO control, status |
| SPI Control | 0x0200 | 0x0200–0x02FF | SPI master, chip-select control |
| I2C Control | 0x0300 | 0x0300–0x03FF | I2C master, device address, data |
| GPIO | 0x0400 | 0x0400–0x04FF | General purpose I/O control |
| PLL / RF Control | 0x0500 | 0x0500–0x05FF | ADF4355, VGA Gain, TRP control |
| Temperature Monitor | 0x0600 | 0x0600–0x06FF | Temp sensor readings, alert threshold |
| Power Monitor | 0x0700 | 0x0700–0x07FF | Voltage/current ADC readings per rail |
| JESD204B Control | 0x0800 | 0x0800–0x08FF | Link config, reset, status |
| Diagnostics | 0x0A00 | 0x0A00–0x0AFF | Fault log, uptime counter, loopback |

### 10.2 Detailed Register Map

**Block 0x0000 — System / Identification**

| Offset | Register Name | Width | R/W | Reset Value | Bit-Field Description |
| --- | --- | --- | --- | --- | --- |
| 0x00 | BOARD_ID | 16 | R | 0xDKF1 | Board identification code |
| 0x01 | FW_VERSION_MAJOR | 16 | R | 0x0001 | Firmware major version |
| 0x02 | FW_VERSION_MINOR | 16 | R | 0x0000 | Firmware minor version |
| 0x03 | SYS_STATUS | 16 | R | 0x0000 | [15:8] Reserved, [7] JESD_LOCKED, [6] TEMP_ALERT, [5] PWR_FAULT, [4] PLL_LOCK, [3:0] INIT_STATE |
| 0x04 | SYS_CTRL | 16 | R/W | 0x0000 | [0] SOFT_RESET, [1] WDT_ENABLE, [2] RF_ENABLE (TRP) |
| 0x05 | IRQ_MASK | 16 | R/W | 0xFFFF | 1=Mask Interrupt. [0] Temp, [1] PWR, [2] JESD |

**Block 0x0100 — UART Control**

| Offset | Register Name | Width | R/W | Reset Value | Bit-Field Description |
| --- | --- | --- | --- | --- | --- |
| 0x00 | BAUD_DIV | 16 | R/W | 0x0007 | Baud rate divisor = FPGA_CLK / (16 × BAUD_RATE) |
| 0x01 | UART_CTRL | 16 | R/W | 0x0001 | [0] UART_ENABLE, [1] LOOPBACK_EN |
| 0x02 | UART_STATUS | 16 | R | 0x0000 | [0] TX_BUSY, [1] RX_AVAIL, [2] RX_OVERRUN |
| 0x03 | TX_DATA | 16 | W | 0x0000 | Write byte to transmit (lower 8 bits) |
| 0x04 | RX_DATA | 16 | R | 0x0000 | Read received byte (lower 8 bits) |

**Block 0x0200 — SPI Control**

| Offset | Register Name | Width | R/W | Reset Value | Bit-Field Description |
| --- | --- | --- | --- | --- | --- |
| 0x00 | SPI_DIV | 16 | R/W | 0x0010 | SCK divider (SCK = CLK / (2*(DIV+1))) |
| 0x01 | SPI_CTRL | 16 | R/W | 0x0000 | [0] CPOL, [1] CPHA, [2] AUTO_SS |
| 0x02 | SPI_TX | 16 | W | 0x0000 | Transmit data |
| 0x03 | SPI_RX | 16 | R | 0x0000 | Receive data |
| 0x04 | SPI_SS | 16 | R/W | 0x000F | Chip Select active low (bit mapped) |

**Block 0x0300 — I2C Control**

| Offset | Register Name | Width | R/W | Reset Value | Bit-Field Description |
| --- | --- | --- | --- | --- | --- |
| 0x00 | I2C_CLK_DIV | 16 | R/W | 0x00C8 | SCL clock divider |
| 0x01 | I2C_CTRL | 16 | R/W | 0x0000 | [0] START, [1] STOP, [2] ACK_EN |
| 0x02 | I2C_TX | 16 | W | 0x0000 | Byte to transmit |
| 0x03 | I2C_RX | 16 | R | 0x0000 | Byte received |
| 0x04 | I2C_STATUS | 16 | R | 0x0000 | [0] BUS_BUSY, [1] RX_DONE, [2] ARB_LOST |

**Block 0x0500 — PLL / RF Control**

| Offset | Register Name | Width | R/W | Reset Value | Bit-Field Description |
| --- | --- | --- | --- | --- | --- |
| 0x00 | VGA_GAIN_3WIRE | 16 | W | 0x0000 | [5:0] 6-bit Gain value (triggers LE pulse) |
| 0x01 | VGA_SPI_CFG | 16 | W | 0x0000 | Triggers SPI write to VGA config register |
| 0x02 | PLL_INT | 16 | W | 0x0040 | ADF4355 Integer Divider |
| 0x03 | PLL_FRAC | 16 | W | 0x0000 | ADF4355 Fractional Divider |
| 0x04 | PLL_MOD | 16 | W | 0x00FF | ADF4355 Modulus |
| 0x05 | PLL_R | 16 | W | 0x0001 | ADF4355 R Divider |
| 0x06 | RF_STATUS | 16 | R | 0x0000 | [0] PLL_LOCK_DET (from ADF4355 MUXout) |

**Block 0x0800 — JESD204B Control**

| Offset | Register Name | Width | R/W | Reset Value | Bit-Field Description |
| --- | --- | --- | --- | --- | --- |
| 0x00 | JESD_CTRL | 16 | R/W | 0x0001 | [0] LINK_ENABLE, [1] RESET_B, [2] SEND_SYSREF |
| 0x01 | JESD_STATUS | 16 | R | 0x0000 | [0] CODE_SYNC, [1] LINK_READY, [2] ALIGN_DONE |
| 0x02 | LANE_RATE | 16 | R/W | 0x000A | Lane rate divider config |
| 0x03 | ERR_COUNT | 16 | R | 0x0000 | Disparity error counter |

**Block 0x0700 — Power Monitor (LTC2992)**

| Offset | Register Name | Width | R/W | Reset Value | Bit-Field Description |
| --- | --- | --- | --- | --- | --- |
| 0x00 | PWR_V12_MSB | 16 | R | 0x0000 | 12V Rail MSB |
| 0x01 | PWR_V12_LSB | 16 | R | 0x0000 | 12V Rail LSB |
| 0x02 | PWR_I12_MSB | 16 | R | 0x0000 | 12V Current MSB |
| 0x03 | PWR_I12_LSB | 16 | R | 0x0000 | 12V Current LSB |
| 0x04 | PWR_V5_MSB | 16 | R | 0x0000 | 5V Rail MSB |
| 0x05 | PWR_V5_LSB | 16 | R | 0x0000 | 5V Rail LSB |

### 10.3 Register Access Rules

**Addressing:** 16-bit address space.

**Word Size:** All registers are 16-bit.

**Read/Write:** Single and Bulk (Burst) supported via UART protocol.

**Atomic Access:** Write to `VGA_GAIN_3WIRE` triggers an immediate hardware state machine update; no separate latch register needed for software.

**Shadow Registers:** PLL registers are double-buffered internally by the ADF4355; write `PLL_CTRL` update bit to latch.

## 11. UART Register Protocol Specification

### 11.1 Physical Layer

**Baud rate:** Default 3.0 Mbps (Configurable via Reg 0x0100).

**Frame format:** 1 start bit, 8 data bits, 1 stop bit, no parity (8N1).

**Physical interface:** 3.3V LVTTL (Level shifter required for RS-422/RS-232 connection).

### 11.2 Command Frame Formats

**Single Register Write (CMD = 0x57 'W'):**

**Single Register Read (CMD = 0x52 'R'):**

**Bulk Register Write (CMD = 0x42 'B'):**

**Bulk Register Read (CMD = 0x62 'b'):**

**Error Response:**

### 11.3 Protocol Timing Constraints

| Parameter | Min | Typical | Max | Unit |
| --- | --- | --- | --- | --- |
| Inter-byte gap (TX side) | — | — | 50 | ms |
| Single Write response time | — | 0.5 | 1 | ms |
| Single Read response time | — | 1 | 2 | ms |
| Bulk Write response time (N=64) | — | 3 | 5 | ms |
| Bulk Read response time (N=64) | — | 3 | 5 | ms |
| Parser reset on timeout | 50 | — | — | ms |

## 12. FPGA Resource Utilization Estimate

| Resource | Available | Estimated Usage | Utilization % |
| --- | --- | --- | --- |
| Slice LUTs | 41,800 | 18,500 | 44% |
| Slice Flip-Flops | 83,600 | 22,000 | 26% |
| Block RAM (36Kb) | 135 | 45 | 33% |
| DSP Slices | 240 | 20 | 8% |
| GTX Transceivers | 4 | 4 | 100% |
| MMCM/PLL | 2 | 2 | 100% |

**Notes:** High Block RAM usage is attributed to the JESD204B RX Buffer and FIFOs. DSP slices are reserved for future Digital Down-Conversion (DDC) implementation; current usage is minimal for data formatting.

## Annexure A — Requirement Traceability Matrix

| S.No. | GLR-ID | Description | Source HRS Section | GLR Section | Verification Method | Status |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | GLR-001 | Serial Communication Interface | HRS §3.2 | 9.1, 11 | Test | Open |
| 2 | GLR-002 | High Speed Communication (JESD) | HRS §3.2 (REQ-HW-006) | 9.2 | Test | Open |
| 3 | GLR-003 | Power Supply Sequencing | HRS §3.3 | 9.3 | Test | Open |
| 4 | GLR-004 | Voltage/Current/Temperature Monitoring | HRS §3.1 | 9.4 | Test | Open |
| 5 | GLR-005 | Flash Interfaces | HRS §3.2 | 9.5 | Test | Open |
| 6 | GLR-006 | RF Chain Control / TRP | HRS §3.1 | 9.6 | Inspection | Open |
| 7 | GLR-007 | VGA Gain Control | HRS §3.1 (REQ-HW-016) | 9.8 | Demonstration | Open |
| 8 | GLR-008 | PLL Tuning Interface | HRS §3.1 (REQ-HW-003) | 9.9 | Test | Open |
| 9 | GLR-009 | Register Address Map | HRS §3.2 | 10 | Inspection | Open |
| 10 | GLR-010 | UART Protocol Specification | HRS §3.2 | 11 | Test | Open |
| 11 | GLR-011 | FPGA Resource Budget | HRS §3.1 | 12 | Analysis | Open |
| 12 | GLR-012 | Input Impedance Matching | HRS §3.3 (REQ-HW-014) | 8 (Netlist) | Test | Open |
| 13 | GLR-013 | EMI Compliance Layout | HRS §3.4 (REQ-HW-011) | 4 (Overview) | Inspection | Open |