# Logical Netlist

## dkfjg

## Block Diagram

**System Architecture Diagram 1**

Diagram 1

## Component Instances

| Ref | Part Number | Component |
| --- | --- | --- |
| U1 | HMC1042LP4BE | Wideband LNA |
| U2 | CBP-1810-LF | Bandpass Filter |
| U3 | HMC698LP4 | Variable Gain Amplifier |
| U4 | HMC774A | RF Mixer |
| U5 | ADA4817 | IF Amplifier |
| U6 | ADC12J4000 | High-Speed ADC |
| U7 | LMK04828 | Clock Generator |
| U8 | ADF4355 | LO Source |
| U9 | LTC2992 | Power Controller |
| U10 | LTM4644 | DC-DC Buck |
| U11 | LT3045 | LDO Regulator 5V |
| U12 | LT3042 | LDO Regulator 3.3V |
| U13 | LT3033 | LDO Regulator 1.8V |
| J1 | SMA-50-EDGE | RF Input Connector |
| J2 | SMA-50-EDGE | LO Input Connector |
| J3 | Samtec-SEARAY | LVDS Output Connector |
| J4 | Molex-43650 | Power Input Connector |
| J5 | header-2x5 | Control Interface |
| L1 | 0402CS-22N | RF Choke 1 |
| L2 | 0402CS-22N | RF Choke 2 |
| L3 | 0402CS-22N | RF Choke 3 |
| L4 | 0402CS-22N | RF Choke 4 |
| L5 | LPS3015-222 | Power Inductor |
| T1 | EQL-5000 | RF Transformer 1 |
| T2 | EQL-5000 | RF Transformer 2 |
| T3 | BALH-0006SM | Balun |
| C1 | 0402NPO-10pF | Input Coupling Cap |
| C2 | 0402NPO-100pF | RF Blocking Cap |
| C3 | 0402NPO-100pF | RF Blocking Cap |
| C4 | 0402NPO-100pF | RF Blocking Cap |
| C5 | 0402NPO-100pF | RF Blocking Cap |
| C6 | 0402X7R-0.1uF | Decoupling Cap |
| C7 | 0402X7R-0.1uF | Decoupling Cap |
| C8 | 0402X7R-0.1uF | Decoupling Cap |
| C9 | 0402X7R-0.1uF | Decoupling Cap |
| C10 | 0402X7R-0.1uF | Decoupling Cap |
| C11 | 0402X7R-0.1uF | Decoupling Cap |
| C12 | 0402X7R-0.1uF | Decoupling Cap |
| C13 | 0402X7R-0.1uF | Decoupling Cap |
| C14 | 0402X7R-0.1uF | Decoupling Cap |
| C15 | 1210X7R-10uF | Bulk Cap |
| C16 | 1210X7R-10uF | Bulk Cap |
| C17 | 1210X7R-10uF | Bulk Cap |
| R1 | 0402-50-1% | Input Termination |
| R2 | 0402-10K-1% | VGPI Resistor |
| R3 | 0402-10K-1% | VGPI Resistor |
| R4 | 0402-4.7K-1% | I2C Pullup |
| R5 | 0402-4.7K-1% | I2C Pullup |
| R6 | 0402-10K-1% | Enable Pullup |
| R7 | 0402-499-1% | Feedback Resistor |
| R8 | 0402-499-1% | Feedback Resistor |

## Pin-to-Pin Connections

| Net | From | Pin | To | Pin | Type |
| --- | --- | --- | --- | --- | --- |
| RF_IN | J1 | 1 | C1 | 1 | RF |
| RF_IN_C | C1 | 2 | U1 | 1 | RF |
| LNA_OUT | U1 | 4 | U2 | 1 | RF |
| FILTER_OUT | U2 | 2 | U3 | 1 | RF |
| VGA_OUT | U3 | 4 | T1 | 1 | RF |
| RF_TO_MIXER | T1 | 2 | U4 | 1 | RF |
| LO_IN | J2 | 1 | U8 | 1 | RF |
| LO_OUT | U8 | 2 | U4 | 3 | RF |
| IF_OUT | U4 | 5 | T2 | 1 | IF |
| IF_TRANSFORMED | T2 | 2 | U5 | 1 | IF |
| IF_AMP_OUT | U5 | 4 | T3 | 1 | IF |
| IF_BALANCED | T3 | 2 | U6 | 1 | IF |
| IF_BALANCED_N | T3 | 3 | U6 | 2 | IF |
| CLK_OUT | U7 | 1 | U6 | 3 | CLOCK |
| CLK_REF | U7 | 2 | U8 | 3 | CLOCK |
| LVDS_D0_P | U6 | 10 | J3 | 1 | LVDS |
| LVDS_D0_N | U6 | 11 | J3 | 2 | LVDS |
| LVDS_D1_P | U6 | 12 | J3 | 3 | LVDS |
| LVDS_D1_N | U6 | 13 | J3 | 4 | LVDS |
| LVDS_CLK_P | U6 | 14 | J3 | 5 | LVDS |
| LVDS_CLK_N | U6 | 15 | J3 | 6 | LVDS |
| I2C_SDA | J5 | 1 | R4 | 1 | DIGITAL |
| I2C_SDA_U3 | R4 | 2 | U3 | 10 | DIGITAL |
| I2C_SDA_U7 | R4 | 2 | U7 | 10 | DIGITAL |
| I2C_SDA_U8 | R4 | 2 | U8 | 10 | DIGITAL |
| I2C_SCL | J5 | 2 | R5 | 1 | DIGITAL |
| I2C_SCL_U3 | R5 | 2 | U3 | 11 | DIGITAL |
| I2C_SCL_U7 | R5 | 2 | U7 | 11 | DIGITAL |
| I2C_SCL_U8 | R5 | 2 | U8 | 11 | DIGITAL |
| VGA_LE | J5 | 3 | U3 | 12 | DIGITAL |
| VGA_CLK | J5 | 4 | U3 | 13 | DIGITAL |
| VGA_DATA | J5 | 5 | U3 | 14 | DIGITAL |
| PWR_12V | J4 | 1 | U10 | 1 | POWER |
| PWR_12V_SW | J4 | 2 | U9 | 1 | POWER |
| BUCK_OUT | U10 | 2 | L5 | 1 | POWER |
| BUCK_SW | L5 | 2 | U10 | 3 | POWER |
| VCC_5V | U11 | 2 | U1 | 5 | POWER |
| VCC_5V_L2 | L2 | 2 | U1 | 6 | POWER |
| VCC_5V_U3 | U11 | 2 | U3 | 5 | POWER |
| VCC_5V_U4 | U11 | 2 | U4 | 6 | POWER |
| VCC_5V_U8 | U11 | 2 | U8 | 5 | POWER |
| VCC_3V3 | U12 | 2 | U5 | 5 | POWER |
| VCC_3V3_U7 | U12 | 2 | U7 | 5 | POWER |
| VCC_1V8 | U13 | 2 | U6 | 16 | POWER |
| GND | J1 | 2 | R1 | 1 | GROUND |
| GND_U1 | U1 | EP | U10 | EP | GROUND |
| GND_U2 | U2 | 3 | U1 | EP | GROUND |
| GND_U3 | U3 | EP | U2 | 3 | GROUND |
| GND_U4 | U4 | EP | U3 | EP | GROUND |
| GND_U5 | U5 | 3 | U4 | EP | GROUND |
| GND_U6 | U6 | EP | U5 | 3 | GROUND |
| GND_U7 | U7 | EP | U6 | EP | GROUND |
| GND_U8 | U8 | EP | U7 | EP | GROUND |
| GND_J4 | J4 | 4 | U8 | EP | GROUND |
| LNA_BYPASS | R2 | 2 | U1 | 8 | ANALOG |
| LNA_VG1 | R3 | 2 | U1 | 7 | ANALOG |
| GND_R2 | R2 | 1 | U1 | EP | GROUND |
| GND_R3 | R3 | 1 | U1 | EP | GROUND |
| FB_U5_P | R7 | 2 | U5 | 2 | ANALOG |
| FB_U5_N | R8 | 2 | U5 | 3 | ANALOG |
| FB_OUT_P | U5 | 2 | R7 | 1 | ANALOG |
| FB_OUT_N | U5 | 3 | R8 | 1 | ANALOG |
| EN_U3 | R6 | 2 | U3 | 6 | DIGITAL |
| VCC_3V3_R6 | R6 | 1 | U12 | 2 | POWER |

## Net Connection List

| Net Name | Reference Designator - Pin No. |
| --- | --- |
| BUCK_OUT | U10 - 2, L5 - 1 |
| BUCK_SW | L5 - 2, U10 - 3 |
| CLK_OUT | U7 - 1, U6 - 3 |
| CLK_REF | U7 - 2, U8 - 3 |
| EN_U3 | R6 - 2, U3 - 6 |
| FB_OUT_N | U5 - 3, R8 - 1 |
| FB_OUT_P | U5 - 2, R7 - 1 |
| FB_U5_N | R8 - 2, U5 - 3 |
| FB_U5_P | R7 - 2, U5 - 2 |
| FILTER_OUT | U2 - 2, U3 - 1 |
| GND | J1 - 2, R1 - 1 |
| GND_J4 | J4 - 4, U8 - EP |
| GND_R2 | R2 - 1, U1 - EP |
| GND_R3 | R3 - 1, U1 - EP |
| GND_U1 | U1 - EP, U10 - EP |
| GND_U2 | U2 - 3, U1 - EP |
| GND_U3 | U3 - EP, U2 - 3 |
| GND_U4 | U4 - EP, U3 - EP |
| GND_U5 | U5 - 3, U4 - EP |
| GND_U6 | U6 - EP, U5 - 3 |
| GND_U7 | U7 - EP, U6 - EP |
| GND_U8 | U8 - EP, U7 - EP |
| I2C_SCL | J5 - 2, R5 - 1 |
| I2C_SCL_U3 | R5 - 2, U3 - 11 |
| I2C_SCL_U7 | R5 - 2, U7 - 11 |
| I2C_SCL_U8 | R5 - 2, U8 - 11 |
| I2C_SDA | J5 - 1, R4 - 1 |
| I2C_SDA_U3 | R4 - 2, U3 - 10 |
| I2C_SDA_U7 | R4 - 2, U7 - 10 |
| I2C_SDA_U8 | R4 - 2, U8 - 10 |
| IF_AMP_OUT | U5 - 4, T3 - 1 |
| IF_BALANCED | T3 - 2, U6 - 1 |
| IF_BALANCED_N | T3 - 3, U6 - 2 |
| IF_OUT | U4 - 5, T2 - 1 |
| IF_TRANSFORMED | T2 - 2, U5 - 1 |
| LNA_BYPASS | R2 - 2, U1 - 8 |
| LNA_OUT | U1 - 4, U2 - 1 |
| LNA_VG1 | R3 - 2, U1 - 7 |
| LO_IN | J2 - 1, U8 - 1 |
| LO_OUT | U8 - 2, U4 - 3 |
| LVDS_CLK_N | U6 - 15, J3 - 6 |
| LVDS_CLK_P | U6 - 14, J3 - 5 |
| LVDS_D0_N | U6 - 11, J3 - 2 |
| LVDS_D0_P | U6 - 10, J3 - 1 |
| LVDS_D1_N | U6 - 13, J3 - 4 |
| LVDS_D1_P | U6 - 12, J3 - 3 |
| PWR_12V | J4 - 1, U10 - 1 |
| PWR_12V_SW | J4 - 2, U9 - 1 |
| RF_IN | J1 - 1, C1 - 1 |
| RF_IN_C | C1 - 2, U1 - 1 |
| RF_TO_MIXER | T1 - 2, U4 - 1 |
| VCC_1V8 | U13 - 2, U6 - 16 |
| VCC_3V3 | U12 - 2, U5 - 5 |
| VCC_3V3_R6 | R6 - 1, U12 - 2 |
| VCC_3V3_U7 | U12 - 2, U7 - 5 |
| VCC_5V | U11 - 2, U1 - 5 |
| VCC_5V_L2 | L2 - 2, U1 - 6 |
| VCC_5V_U3 | U11 - 2, U3 - 5 |
| VCC_5V_U4 | U11 - 2, U4 - 6 |
| VCC_5V_U8 | U11 - 2, U8 - 5 |
| VGA_CLK | J5 - 4, U3 - 13 |
| VGA_DATA | J5 - 5, U3 - 14 |
| VGA_LE | J5 - 3, U3 - 12 |
| VGA_OUT | U3 - 4, T1 - 1 |

## Validation Notes

CRITICAL: HMC698LP4 VGA operating temperature is -40 to +85°C. This does NOT meet the military temperature requirement of -55 to +125°C. Must select extended temperature range variant or alternative component.

CRITICAL: HMC774A mixer operating temperature is -40 to +85°C (standard grade). Extended temperature version or alternative mixer required for -55 to +125°C operation.

CRITICAL: ADC12J4000 available temperature grades: -40 to +85°C (commercial) or -40 to +105°C (industrial). Does NOT meet -55°C lower limit. Must verify military-grade availability or select alternative.

WARNING: System gain calculation: LNA (24dB) + Filter (-3.5dB) + VGA (22dB max) + Mixer (-7.5dB) + IF Amp (12dB) = 47dB max. Exceeds 30-40dB requirement - may need gain reduction for stability.

WARNING: System noise figure cascade: Stage 1 LNA (2.5dB) sets NF close to requirement. Filter loss (3.5dB) after LNA degrades NF. Consider filter placement before LNA or lower-loss filter.

WARNING: Power budget estimated at 8.5W: LNA (180mW) + VGA (350mW) + Mixer (450mW) + IF Amp (100mW) + LO (800mW) + ADC (2.5W) + Clock (1.2W) + Regulators (2W). Within 5-10W requirement but at upper end.

INFO: LVDS output interface from ADC requires careful impedance matching (100 ohm differential) and ESD protection for military EMI compliance.

INFO: JESD204B interface from ADC requires FPGA/controller with JESD204B IP core. Not specified in design - may need external FPGA or processor.

INFO: LO input via ADF4355 PLL provides flexible frequency synthesis. Need to specify reference clock input source and frequency for PLL configuration.

INFO: Multiple voltage rails (5V, 3.3V, 1.8V) generated from single 12V input. Ensure proper sequencing for ADC (typically 1.8V core before 3.3V I/O).

INFO: I2C control bus shared between VGA, clock generator, and PLL. Ensure address conflicts resolved and pullup resistors (4.7K) provided for open-drain signals.

RECOMMENDATION: Add EMI/ESD protection on RF input (J1) and LVDS output (J3) for MIL-STD-461 compliance.

RECOMMENDATION: Consider adding temperature sensors and compensation circuitry for wide temperature range operation.

RECOMMENDATION: Add ESD protection diodes on all external interfaces (RF_IN, LO_IN, LVDS_OUT, CTRL, PWR_IN).