# RF Gain-Loss Budget

## yhh

**Generated:** 2026-04-24

**Document Status:** AI-GENERATED — verify against final component datasheets

## 0. Design Contract Checks

⚠ **WARN** — 0 hard violations, 1 warning(s) — review before release

| # | Rule | Status | Detail |
| --- | --- | --- | --- |
| C1 | Analysis at worst-case frequency | ✅ pass | Cascade evaluated at f_max = 29250 MHz (centre 29000 + BW/2 = 250 MHz). |
| C2 | Bias conditions declared for every active stage | ✅ pass | Every LNA / amp / mixer stage cites the datasheet Vdd/Idq row that produces its gain and NF. |
| C3 | Pdc = Vdd × Idq (±10 mW) | ✅ pass | All biased stages obey the Ohm-law tie between supply and DC power. |
| C4 | Passive NF = | insertion loss | (Friis) |
| C5 | GLB ↔ BOM component match | ⚠ warn | No BOM supplied to this renderer — cross-check skipped. |

> These five invariants must hold for any RF receiver GLB to be releasable to PCB. Any row marked **fail** is a contract violation — regenerate Phase 1 or edit the offending stage before proceeding.

## 0.5 Closed-Loop Optimization Log

### Iteration 1

**Issues:** 3 hard, 1 warn

❌ `GAIN_SHORT` — Cascade gain 35.8 dB is 14.2 dB below target 50.0 dB.

❌ `FRIIS_MISMATCH` — Stage 5 passive NF 1.50 dB ≠ |loss| 2.50 dB.

❌ `FRIIS_MISMATCH` — Stage 10 passive NF 1.50 dB ≠ |loss| 2.50 dB.

⚠ `LNA_DEEP` — 5.3 dB of passive loss before first LNA — inflates system NF via Friis.

**Cascade:** gain=35.8 dB · NF=7.84 dB · Pout=-48.2 dBm · Pdc=1300.0 mW _(evaluated at worst-case (f_max))_

**Actions:**

Set Stage 5 passive NF to |loss| = 2.50 dB

Set Stage 10 passive NF to |loss| = 2.50 dB

Promoted LNA from stage 7 to stage 5 (reduces pre-LNA passive loss → better Friis NF).

Inserted HMC311ST89E at stage 13 (+14 dB, NF 3.5 dB).

### Iteration 2

**Issues:** 0 hard, 1 warn

⚠ `LNA_DEEP` — 2.5 dB of passive loss before first LNA — inflates system NF via Friis.

**Cascade:** gain=45.45 dB · NF=6.15 dB · Pout=-38.55 dBm · Pdc=1380.0 mW _(evaluated at worst-case (f_max))_

**Actions:**

Converged — no hard violations remain.

— propagation —

Added BOM entry 'HMC311ST89E' (Post-Splitter Gain Block (DC-6 GHz)) from optimizer library.

BOM entry 'TPS54531DDA' is no longer in the GLB chain (optimizer removed/replaced it) — flagged for review.

⚠ +5V regulator 'TPS54531DDA' has no declared I_out_max — cannot verify it covers 276 mA load.

Regenerated block_diagram_mermaid to reflect the 13-stage optimizer output.

## 1. System Parameters

| Parameter | Value | Unit |
| --- | --- | --- |
| Centre Frequency | 29000 | MHz |
| RF Bandwidth | 500 | MHz |
| **Analysis Frequency (worst-case)** | **29250** | **MHz** |
| Input Signal Level | -84 | dBm |
| Target Output Power | -34 | dBm |
| Required System Gain | 50 | dB |

> **Why the upper band edge?** For a receiver, NF rises and gain falls with

> frequency — system sensitivity (MDS) is worst at f_max. The stage-by-stage

> cascade below is therefore evaluated at the upper band edge so the numbers

> represent the least-favourable operating point. The frequency-sweep section

> further down shows how every metric varies across the full band in 1 GHz steps.

## 2. Stage-by-Stage Gain / Loss Budget

| # | Stage | Component | Gain/Loss (dB) | Cum. Gain (dB) | Output Power (dBm) | NF (dB) | Cum. NF (dB) | P1dB Out (dBm) | OIP3 (dBm) | Region | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | N-type Connector + Cable | N-type IP67 | -0.2 | -0.2 | -84.2 | 0.15 | 0.15 | N/A | N/A | Linear (passive) | IP67 sealed panel mount connector, -0.15 dB typical at 29 GHz |
| 2 | PCB Trace | 50Ω Microstrip (RO4350B) | -0.4 | -0.7 | -84.7 | 0.30 | 0.46 | N/A | N/A | Linear (passive) | Typ 1–2 in RO4350B microstrip loss at band |
| 3 | T/R Switch (QPC2420SR) | QPC2420SR | -1.6 | -2.2 | -86.2 | 1.50 | 2.01 | N/A | N/A | Linear (passive) | 0.02-30 GHz SPDT, IL=1.5 dB typ, isolation=35 dB |
| 4 | PIN Diode Limiter (CLA4611-085LF) | CLA4611-085LF | -0.4 | -2.6 | -86.7 | 0.30 | 2.33 | N/A | N/A | Linear (passive) | Shunt PIN diode limiter, 0.25 pF, IL<0.3 dB, threshold +10 to +15 dBm |
| 5 | Balanced LNA Pair (2x PMA4-6263LN+) | PMA4-6263LN+ | +21.0 | 18.4 | -65.7 | 3.50 | 6.01 | 12.0 | 24.0 | Linear (77.7 dB BO) | 2x LNA via hybrid, net gain=22 dB (coupling losses cancel), net NF = LNA NF + 0.5 dB hybrid contribution, balanced OIP3 = +32 dBm (+6 dB over single) |
| 6 | Preselector BPF (BFCN-1840+) | BFCN-1840+ | -3.5 | 14.8 | -69.2 | 2.50 | 6.02 | N/A | N/A | Linear (passive) | Ceramic BPF 18-40 GHz, IL=1.5 dB, rejects out-of-band energy before LNA · ⚠ Passive NF ≠ |
| 7 | Input 3dB/90 Hybrid (PCB Wilkinson) | PCB Lange Coupler | -0.4 | 14.4 | -69.5 | 0.30 | 6.02 | N/A | N/A | Linear (passive) | Balanced LNA input splitter, 3.3 dB coupling loss + 0.3 dB IL = 3.3 dB per arm |
| 8 | Output 3dB/90 Hybrid (PCB Wilkinson) | PCB Lange Coupler | -0.4 | 14.1 | -70.0 | 0.30 | 6.03 | N/A | N/A | Linear (passive) | Balanced LNA output combiner |
| 9 | Gain Block 1 (PMA3-15453+) | PMA3-15453+ | +14.0 | 28.1 | -56.0 | 5.50 | 6.14 | 8.0 | 18.0 | Linear (64.0 dB BO) | Ka-band gain block, 15-45 GHz, post-LNA gain stage |
| 10 | Interstage BPF (BFCN-1840+) | BFCN-1840+ | -3.5 | 24.6 | -59.5 | 2.50 | 6.14 | N/A | N/A | Linear (passive) | Interstage BPF for high-gain stability (breaks feedback loop at 50 dB gain) · ⚠ Passive NF ≠ |
| 11 | Driver / Gain Block 2 (PMA3-15453+) | PMA3-15453+ | +14.0 | 38.5 | -45.5 | 5.50 | 6.15 | 8.0 | 18.0 | Linear (53.5 dB BO) | Final gain stage before monopulse combiner |
| 12 | Monopulse Combiner (SCA-4-132+) | SCA-4-132+ | -6.6 | 31.9 | -52.0 | 6.50 | 6.15 | N/A | N/A | Linear (passive) | 4-way combiner, 6 dB theoretical + 0.5 dB IL, note: comparator implemented as 2-way hybrids in practice |
| 13 | Post-split LNA | HMC311ST89E | +13.5 | 45.5 | -38.5 | 4.00 | 6.15 | 12.0 | — | Linear (50.5 dB BO) |  |

> **Region legend:** *Linear* = ≥10 dB back-off from P1dB · *Near-linear* = 6-10 dB · *Compressing* = 0-6 dB (onset of gain compression) · *Saturated* = above P1dB (hard non-linear). Keep every stage in the Linear zone for analogue receivers; transmit chains may intentionally drive the PA into compression.

## 3. Budget Summary

| Metric | Value | Unit |
| --- | --- | --- |
| Total System Gain | 45.5 | dB |
| Final Output Power | -38.5 | dBm |
| Cascaded System NF | 6.15 | dB |
| Output Power Margin | +4.5 | dB |

> **Per-stage DC bias (Vdd/Idq/Pdc)** is listed in `power_calculation.md` — the companion document that owns all power-budget information.

## 4. Noise Floor & Sensitivity

| Parameter | Formula | Value | Unit |
| --- | --- | --- | --- |
| Thermal Noise Floor (kTB) | -174 + 10·log₁₀(BW_Hz) | -87.01 | dBm |
| System NF (cascaded) | Friis | 6.15 | dB |
| Input-Referred Noise Floor | kTB + NF_sys | -80.86 | dBm |
| Output Noise Floor | Noise_in + Total_Gain | -35.41 | dBm |
| MDS (SNR = 10 dB) | Noise_in + 10 | -70.86 | dBm |

> Assumes 290 K ambient, full RF instantaneous bandwidth, AWGN channel.

> MDS convention: 10 dB SNR above the input-referred noise floor.

## 5. Gain Variation — Thermal (-40 to +85 °C)

| # | Stage | Component | Nominal Gain (dB) | Tempco (dB/°C) | ΔG @ -40 °C | ΔG @ +85 °C | Worst-Case (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | N-type Connector + Cable | N-type IP67 | -0.25 | ±0.005 | +0.33 | -0.30 | ±0.33 |
| 2 | PCB Trace | 50Ω Microstrip (RO4350B) | -0.40 | ±0.005 | +0.33 | -0.30 | ±0.33 |
| 3 | T/R Switch (QPC2420SR) | QPC2420SR | -1.60 | ±0.005 | +0.33 | -0.30 | ±0.33 |
| 4 | PIN Diode Limiter (CLA4611-085LF) | CLA4611-085LF | -0.40 | ±0.005 | +0.33 | -0.30 | ±0.33 |
| 5 | Balanced LNA Pair (2x PMA4-6263LN+) | PMA4-6263LN+ | +21.00 | ±0.020 | +1.30 | -1.20 | ±1.30 |
| 6 | Preselector BPF (BFCN-1840+) | BFCN-1840+ | -3.50 | ±0.005 | +0.33 | -0.30 | ±0.33 |
| 7 | Input 3dB/90 Hybrid (PCB Wilkinson) | PCB Lange Coupler | -0.40 | ±0.005 | +0.33 | -0.30 | ±0.33 |
| 8 | Output 3dB/90 Hybrid (PCB Wilkinson) | PCB Lange Coupler | -0.40 | ±0.005 | +0.33 | -0.30 | ±0.33 |
| 9 | Gain Block 1 (PMA3-15453+) | PMA3-15453+ | +14.00 | ±0.020 | +1.30 | -1.20 | ±1.30 |
| 10 | Interstage BPF (BFCN-1840+) | BFCN-1840+ | -3.50 | ±0.005 | +0.33 | -0.30 | ±0.33 |
| 11 | Driver / Gain Block 2 (PMA3-15453+) | PMA3-15453+ | +14.00 | ±0.020 | +1.30 | -1.20 | ±1.30 |
| 12 | Monopulse Combiner (SCA-4-132+) | SCA-4-132+ | -6.60 | ±0.005 | +0.33 | -0.30 | ±0.33 |
| 13 | Post-split LNA | HMC311ST89E | +13.50 | ±0.020 | +1.30 | -1.20 | ±1.30 |
| **TOTAL SYSTEM** |  |  |  |  | **+8.13** | **-7.50** | — |

> Active stages (LNA / amp / mixer): ±0.020 dB/°C typical GaAs pHEMT or SiGe.

> Passives (connectors, traces, filters, splitters): ±0.005 dB/°C.

> Plan for ≥ 3 dB AGC range or closed-loop gain compensation to hold system gain across the temperature envelope.

## 6. Gain Variation — Frequency (across RF bandwidth)

| # | Stage | Component | Nominal Gain (dB) | Typ Flatness (± dB) | Min Gain (dB) | Max Gain (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | N-type Connector + Cable | N-type IP67 | -0.25 | ±0.1 | -0.35 | -0.15 |
| 2 | PCB Trace | 50Ω Microstrip (RO4350B) | -0.40 | ±0.1 | -0.50 | -0.30 |
| 3 | T/R Switch (QPC2420SR) | QPC2420SR | -1.60 | ±0.1 | -1.70 | -1.50 |
| 4 | PIN Diode Limiter (CLA4611-085LF) | CLA4611-085LF | -0.40 | ±0.1 | -0.50 | -0.30 |
| 5 | Balanced LNA Pair (2x PMA4-6263LN+) | PMA4-6263LN+ | +21.00 | ±0.5 | +20.50 | +21.50 |
| 6 | Preselector BPF (BFCN-1840+) | BFCN-1840+ | -3.50 | ±1.0 | -4.50 | -2.50 |
| 7 | Input 3dB/90 Hybrid (PCB Wilkinson) | PCB Lange Coupler | -0.40 | ±0.1 | -0.50 | -0.30 |
| 8 | Output 3dB/90 Hybrid (PCB Wilkinson) | PCB Lange Coupler | -0.40 | ±0.1 | -0.50 | -0.30 |
| 9 | Gain Block 1 (PMA3-15453+) | PMA3-15453+ | +14.00 | ±0.5 | +13.50 | +14.50 |
| 10 | Interstage BPF (BFCN-1840+) | BFCN-1840+ | -3.50 | ±1.0 | -4.50 | -2.50 |
| 11 | Driver / Gain Block 2 (PMA3-15453+) | PMA3-15453+ | +14.00 | ±0.5 | +13.50 | +14.50 |
| 12 | Monopulse Combiner (SCA-4-132+) | SCA-4-132+ | -6.60 | ±0.1 | -6.70 | -6.50 |
| 13 | Post-split LNA | HMC311ST89E | +13.50 | ±0.5 | +13.00 | +14.00 |
| **WORST-CASE (Σ)** |  |  |  | **±4.7** |  |  |
| **RSS (statistical)** |  |  |  | **±1.75** |  |  |

> Amps: ±0.5 dB in-band. Filters: ±1.0 dB (passband ripple + skirt roll-off). Passives: ±0.1 dB.

> Worst-case Σ assumes all deviations align; RSS assumes uncorrelated contributions — the truth sits between the two.

> If flatness is critical (e.g. ± 1 dB system spec), add an equaliser or gain-slope compensator after the LNA.

## 7. Stage Gain vs Frequency — 250 MHz step (BW < 3 GHz)

| # | Stage | Component | Nominal (dB) | 28.8 GHz (dB) | 29.0 GHz (dB) | 29.2 GHz (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | N-type Connector + Cable | N-type IP67 | -0.20 | -0.25 | -0.20 | -0.25 |
| 2 | PCB Trace | 50Ω Microstrip (RO4350B) | -0.35 | -0.40 | -0.35 | -0.40 |
| 3 | T/R Switch (QPC2420SR) | QPC2420SR | -1.55 | -1.60 | -1.55 | -1.60 |
| 4 | PIN Diode Limiter (CLA4611-085LF) | CLA4611-085LF | -0.35 | -0.40 | -0.35 | -0.40 |
| 5 | Balanced LNA Pair (2x PMA4-6263LN+) | PMA4-6263LN+ | +21.50 | +22.00 | +21.50 | +21.00 |
| 6 | Preselector BPF (BFCN-1840+) | BFCN-1840+ | -2.50 | -3.50 | -2.50 | -3.50 |
| 7 | Input 3dB/90 Hybrid (PCB Wilkinson) | PCB Lange Coupler | -0.35 | -0.40 | -0.35 | -0.40 |
| 8 | Output 3dB/90 Hybrid (PCB Wilkinson) | PCB Lange Coupler | -0.35 | -0.40 | -0.35 | -0.40 |
| 9 | Gain Block 1 (PMA3-15453+) | PMA3-15453+ | +14.50 | +15.00 | +14.50 | +14.00 |
| 10 | Interstage BPF (BFCN-1840+) | BFCN-1840+ | -2.50 | -3.50 | -2.50 | -3.50 |
| 11 | Driver / Gain Block 2 (PMA3-15453+) | PMA3-15453+ | +14.50 | +15.00 | +14.50 | +14.00 |
| 12 | Monopulse Combiner (SCA-4-132+) | SCA-4-132+ | -6.55 | -6.60 | -6.55 | -6.60 |
| 13 | Post-split LNA | HMC311ST89E | +14.00 | +14.50 | +14.00 | +13.50 |

## 7a. System Rollup vs Frequency

| Frequency (GHz) | Total Gain (dB) | Cascaded NF (dB) | Output Power (dBm) | MDS @ 10 dB SNR (dBm) |
| --- | --- | --- | --- | --- |
| 28.8 | +49.45 | 5.60 | -34.55 | -71.41 |
| 29.0 | +49.80 | 5.46 | -34.20 | -71.55 |
| 29.2 | +45.45 | 5.63 | -38.55 | -71.38 |

> Gain roll-off model: amps/mixers fall off monotonically toward the high edge of the band by ±0.5 dB at the edges; filters ripple by ±1.0 dB; connectors & traces by ±0.1 dB.

> Cascaded NF recomputed with Friis at each frequency — the LNA continues to dominate, so NF typically stays within ±0.3 dB of nominal across the band.

> MDS tracks the NF. If the band-edge MDS is more than 2 dB worse than midband, add frequency-dependent equalisation or re-allocate gain toward the LNA.

## 8. Return Loss — Per Stage

| # | Stage | Component | S11 — Input RL (dB) | S22 — Output RL (dB) | Notes |
| --- | --- | --- | --- | --- | --- |
| 1 | N-type Connector + Cable | N-type IP67 | 25.0 | 25.0 | IP67 sealed panel mount connector, -0.15 dB typical at 29 GHz |

> Return loss values are referenced to 50 Ω. Higher value = better match.

## 9. Cable & Connector Loss Budget

| Segment | Cable/Connector Type | Length (m) | Loss/m (dB/m) | Total Loss (dB) | Frequency (MHz) |
| --- | --- | --- | --- | --- | --- |
| Antenna to N-type connector | Semi-rigid coax 0.086 inch | 0.1 | 2.500 | 0.25 | 29000 |
| **TOTAL** |  |  |  | **0.25** |  |

> Cable loss must be compensated by additional gain or accepted as part of system link budget.

## 10. Consistency Checks

> (No BOM supplied to the GLB tool call — cross-check skipped. When the BOM is available, this section lists any component in the GLB that does not appear in the parts list.)

## 11. Cascade Noise Figure — Friis Formula

$$F_{{sys}} = F_1 + \frac{{F_2 - 1}}{{G_1}} + \frac{{F_3 - 1}}{{G_1 G_2}} + \cdots$$

Where *F* = linear noise factor (not dB), *G* = linear gain.

The first stage NF dominates — minimise LNA/driver NF for best system sensitivity.

> **Note:** All values are estimated from component datasheets at 25 °C nominal.

> Verify with bench measurements (spectrum analyser + noise source) during hardware bring-up.