# Component Recommendations

## rx band

### 1. RF LNA — first-stage amplifier at the antenna input for each of the 4 channels (GaN technology preferred)

Primary Choice: ZVA-183WA-S+ (Mini-Circuits)

RF Amplifier Gain Block, 100 - 18000 MHz, 50 Ohm — used here as the wideband front-end amplifier (closest match to 18-40 GHz from distributor pool)

🔗 DigiKey 🔗 Mouser

Distributor data: source: mouser

| Spec | Value |
| --- | --- |
| Frequency | 0.1-18 GHz |
| Gain | 22 dB typ |
| Noise_Figure | ~3.5 dB est |
| P1dB | +16 dBm typ |
| OIP3 | ~+26 dBm est |
| Package | SMT |
| Supply | 5V |

Alternatives: - PMA2-123LNW+ (Mini-Circuits): Wider bandwidth to 12 GHz, lower NF (~2 dB), but lower gain (~15 dB) and lower P1dB - QPL1000SR (Qorvo): GaN technology as specified, but limited to 8-11 GHz band — not suitable for 18-40 GHz without redesign

Selection Rationale: Only wideband SMT amplifier from pool covering up to 18 GHz. Design note: for the 18-40 GHz band, a 40+ GHz GaN LNA (e.g. Northrop Grumman ALH369 or custom MMIC) will be needed at final PDR.

### 2. RF1 Mixer — first downconverter from RF (18-32 GHz) to IF1 (~4 GHz) for each channel

Primary Choice: CMD180C3 (Qorvo)

RF Mixer 18-32 GHz Double Balanced Mixer — covers the lower portion of the required RF band

🔗 DigiKey 🔗 Mouser

Distributor data: source: mouser

| Spec | Value |
| --- | --- |
| RF_Frequency | 18-32 GHz |
| IF_Frequency | DC-8 GHz |
| LO_Frequency | 14-36 GHz |
| Conversion_Loss | 8 dB typ |
| LO_Rdrive | +13 dBm |
| Isolation_LO_RF | 35 dB |
| Package | SMT |
| Supply | Passive |

Alternatives: - CMD181 (Qorvo): Covers 26-45 GHz extending to 40 GHz, but NRND lifecycle status. Use for upper sub-band.

Selection Rationale: Only pool mixer covering the 18+ GHz band. CMD180C3 covers 18-32 GHz; CMD181 (26-45 GHz, NRND) covers the upper end. Two mixer types needed for full 18-40 GHz coverage. Selected CMD180C3 as primary for 18-32 GHz.

### 3. RF2 Mixer — second downconverter from IF1 (~4 GHz) to IF2 (~500 MHz) for each channel

Primary Choice: MMIQ-0205HSM-2 (Marki Microwave)

Miniaturized surface-mount multi-octave 1.75-5.0 GHz IQ mixer, double balanced

🔗 DigiKey 🔗 Mouser

Distributor data: source: mouser

| Spec | Value |
| --- | --- |
| RF_Frequency | 1.75-5.0 GHz |
| LO_Frequency | 1.75-5.0 GHz |
| IF_Frequency | DC-3 GHz |
| Conversion_Loss | 8 dB typ |
| LO_Drive | +13 dBm |
| Image_Rejection | 25 dB typ |
| Package | SMT |

Alternatives: - CMD177C3-EVB (Qorvo): Double balanced mixer 6-14 GHz, wider band but no IQ capability — requires external image-reject filter

Selection Rationale: IQ mixer provides inherent image rejection for the 2nd IF conversion, critical for selectivity. Covers the IF1 frequency range around 4 GHz.

### 4. LO1 PLL Synthesizer — generates LO1 for first downconversion (14-36 GHz)

Primary Choice: LMX2820RTCT (Texas Instruments)

22.6-GHz wideband RF synthesizer with integrated VCO and phase detector

🔗 DigiKey 🔗 Mouser

Distributor data: source: mouser

| Spec | Value |
| --- | --- |
| Output_Frequency | 0.01-22.6 GHz (with divider to 43 GHz) |
| Phase_Noise_10kHz | -120 dBc/Hz typ |
| Reference_Input | 10-500 MHz |
| Output_Power | +7 dBm typ |
| Interface | SPI |
| Package | WQFN-24 |
| Supply | 3.3V |

Alternatives: - LMX2595RHAT (Texas Instruments): 20 GHz max output, slightly lower frequency coverage but proven reliability and lower cost

Selection Rationale: Highest-frequency PLL from pool. 22.6 GHz output frequency with output divider extends effective range to cover the LO1 band. Excellent phase noise performance.

### 5. LO2 PLL Synthesizer — generates LO2 for second downconversion (~3.5 GHz)

Primary Choice: ADF4383BCCZ (Analog Devices)

PLL/VCO, 656 MHz to 21 GHz — used for LO2 generation around 3.5 GHz

🔗 DigiKey 🔗 Mouser

Distributor data: source: mouser

| Spec | Value |
| --- | --- |
| Output_Frequency | 0.656-21 GHz |
| Phase_Noise | Ultra-low |
| Reference_Input | 10-500 MHz |
| Interface | SPI |
| Package | LCC-24 |
| Supply | 3.3V |

Alternatives: - LMX2592RHAT (Texas Instruments): 9.8 GHz max, sufficient for LO2, lower cost

Selection Rationale: Wideband PLL covers the LO2 frequency range. From ADI, complementary to the TI LO1 synth, providing diverse supply chain.

### 6. Reference OCXO — 10 MHz oven-controlled crystal oscillator for LO phase noise floor

Primary Choice: KOVTL10MDBFBCB (KYOCERA AVX)

14-pin DIP standard OCXO, 10 MHz, SC-cut crystal, ultra-low phase noise

🔗 DigiKey 🔗 Mouser

Distributor data: source: mouser

| Spec | Value |
| --- | --- |
| Frequency | 10 MHz |
| Stability | 1e-10 (aging/day) |
| Phase_Noise_10kHz | -160 dBc/Hz typ |
| Supply | 5V or 12V |
| Package | 14-pin DIP |
| Operating_Temp | -40 to +85C |

Alternatives: - OSJ7014-10.0M (Pletronics Inc.): 3.3V supply, SC-cut, slightly different phase noise profile

Selection Rationale: Lowest phase noise OCXO from pool. SC-cut crystal provides excellent short-term stability critical for radar coherent processing.

### 7. ADC — 16-bit digitiser for final IF per channel

Primary Choice: LTC2107IUK#PBF (Analog Devices)

16-Bit, 210 Msps High Performance ADC with LVDS outputs

🔗 DigiKey 🔗 Mouser

Distributor data: source: mouser

| Spec | Value |
| --- | --- |
| Resolution | 16-bit |
| Sample_Rate | 210 Msps |
| SNR | 76.5 dBFS typ |
| SFDR | 89 dBc typ |
| Input_Bandwidth | 700 MHz |
| Interface | LVDS |
| Supply | 1.8V core, 3.3V IO |
| Package | QFN-64 |

Alternatives: - LTC2184IUP#PBF (Analog Devices): 16-bit dual ADC at 105 Msps — lower sample rate per channel but integrates 2 ADCs in one package, reducing BOM count

Selection Rationale: Only 16-bit high-speed ADC from pool with LVDS interface. 210 Msps is adequate for 500 MHz IBW in under-sampled or baseband IF configurations. SFDR of 89 dBc exceeds the 80 dB spec.

### 8. FPGA — digital signal processing, DDC, pulse compression, coherent integration for all 4 channels

Primary Choice: XC7K355T-1FFG901I (AMD)

Kintex-7 FPGA, 355K logic cells, 300 I/O, 901-pin FCBGA, industrial temperature grade

🔗 DigiKey

Distributor data: source: digikey

| Spec | Value |
| --- | --- |
| Logic_Cells | 355,360 |
| DSP_Slices | 840 |
| BRAM | 16,120 Kb |
| I_O | 300 |
| Package | FFG901 |
| Speed_Grade | -1 (industrial) |
| Supply | 1.0V core, 1.8V/2.5V/3.3V bank |

Alternatives: - XC7K420T-1FFG901C (AMD): Larger device (420K LC, 1540 BRAM) for heavier processing, but commercial temp grade only

Selection Rationale: Kintex-7 as specified by user. -1 speed grade industrial temp version for -55 to 125C support. 355K logic cells and 840 DSP slices sufficient for 4-channel DDC and pulse compression.

### 9. IF Gain Block — inter-stage amplifier at IF1 (~4 GHz) between the two mixer stages for gain makeup

Primary Choice: PMA2-123LNW+ (Mini-Circuits)

SMT MMIC Low Noise Linear Gain Block Amplifier, 0.01-12 GHz, 50 Ohm, 2x2 mm QFN

🔗 DigiKey 🔗 Mouser

Distributor data: source: mouser

| Spec | Value |
| --- | --- |
| Frequency | 0.01-12 GHz |
| Gain | 22 dB typ at 4 GHz |
| Noise_Figure | ~2.5 dB est at 4 GHz |
| P1dB | ~+15 dBm est |
| OIP3 | ~+26 dBm est |
| Package | 2x2 mm QFN |
| Supply | 5V, 90 mA |

Alternatives: - ZVA-183WA-S+ (Mini-Circuits): Same as LNA — higher power but larger package, could simplify BOM by using same part

Selection Rationale: Wideband gain block covering IF1 at 4 GHz. Provides gain between the two mixer stages to overcome mixer conversion loss while maintaining good NF. SMT QFN package suitable for compact layout.

### 10. VGA / AGC — voltage variable attenuator for automatic gain control in the IF chain

Primary Choice: TGL2767-SMEVB (Qorvo)

2-31 GHz Voltage Variable Attenuator evaluation board — used for AGC range control

🔗 DigiKey 🔗 Mouser

Distributor data: source: mouser

| Spec | Value |
| --- | --- |
| Frequency | 2-31 GHz |
| Attenuation_Range | 0-20 dB typ |
| Insertion_Loss | 2.5 dB typ |
| Control_Voltage | 0 to -3V |
| Package | Evaluation board (die available) |
| Supply | Passive (control voltage only) |

Alternatives: - MAAT-010521-L1TR05 (MACOM): 5-16 GHz range, SMT 3mm QFN package — better suited for IF placement but limited frequency range

Selection Rationale: Only VVA from pool covering millimeter-wave frequencies. Provides AGC range to keep ADC in linear region. Design note: for IF2 placement, a lower-frequency VVA may be more appropriate at PDR.

### 11. 5V LDO Regulator — low-noise 5V supply rail from +15V input for RF amplifiers and mixers

Primary Choice: LM2940S-5.0/NOPB (Texas Instruments)

1A Low-Dropout Regulator, 5.0V fixed output, designed for automotive/military applications

🔗 DigiKey 🔗 Mouser

Distributor data: source: mouser

| Spec | Value |
| --- | --- |
| Output_Voltage | 5.0V fixed |
| Max_Output_Current | 1A |
| Dropout_Voltage | 0.5V typ at 1A |
| PSRR | 72 dB at 120 Hz |
| Noise | ~100 uV rms |
| Package | TO-263 (SMD) |
| Operating_Temp | -40 to 125C |

Alternatives: - ZLDO1117QK50TC (Diodes Incorporated): Lower cost, 800 mA output, automotive qualified — may need paralleling for higher current

Selection Rationale: Robust 1A LDO with good PSRR for RF supply rails. LM2940 series is widely used in military applications. Low noise critical for RF amplifier supply decoupling.

### 12. 3.3V LDO Regulator — 3.3V digital supply rail for FPGA bank I/O, PLLs, and ADC digital I/O

Primary Choice: MCP1826S-5002E/DBVAO (Microchip Technology)

1.0A Low Voltage Low Quiescent Current LDO Regulator, 5.0V input to 3.3V output capable

🔗 DigiKey 🔗 Mouser

Distributor data: source: mouser

| Spec | Value |
| --- | --- |
| Output_Voltage | 3.3V (adjustable) |
| Max_Output_Current | 1A |
| Dropout_Voltage | 210 mV typ |
| PSRR | 70 dB at 1 kHz |
| Noise | 50 uV rms typ |
| Package | SOT-223 |
| Supply | Up to 6V input |

Alternatives: - MCP1726-ADJE/MFVAO (Microchip Technology): Similar specs, 1A, adjustable, different package (DFN-8) — pin-compatible alternative

Selection Rationale: 1A LDO with very low noise for sensitive digital rails. Adjustable output can be set to 3.3V via external resistor divider.